The following specification text has been prepared to assist design professionals in the preparation of a specification section incorporating EndurisTM 3525 used as a remedial roof coating system applied over existing metal roofing. Design professional is responsible for editing information to meet their Project requirements and coordinate with other specifications sections and Drawings.

SECTION 07 56 00

FLUID APPLIED ROOFING

Editor notes are included in hidden text to assist with product attributes and performance.

EndurisTM 3525 forms a durable, breathable, and waterproof roofing membrane that is highly resistant to degradation from heat, cold, UV light, and natural weathering.

EndurisTM 3525 is not recommended for pedestrian, deck, or frequent traffic bearing applications.

For assistance on the use of the products in this section, contact Momentive Performance Materials at 877-943-7325, or visit their website at www.siliconesforbuilding.com.

GENERAL

SUMMARY

Section Includes: Remedial roof coating system applied over an existing metal roof system.

Related Sections:

Section 07 01 50 – Roof Repairs.

Section 07 62 00 – Sheet Metal Flashing and Trim.

Section 07 71 00 – Roof Specialties.

Section 07 72 00 – Roof Accessories.

Section 07 92 00 – Joint Sealants.

SUBMITTALS

Action Submittals

Product Data: Manufacturer’s current technical data sheet, descriptive data and application instructions.

Shop Drawings: Roof plan showing area of Work and roof details.

Samples: Physical samples and color samples of products to be installed.

Adhesion test results.

Informational Submittals:

Retain Installer Certification for Projects requiring a Product Plus or Labor and Material Warranty.

Certifications: Signed by roof coating manufacturer certifying that installer is approved, authorized, or licensed by manufacturer to install coating system

Sample Warranty:

Installer’s warranty sample.

Manufacturer’s warranty sample.

Manufacturer’s Field Installation Guide.

QUALITY ASSURANCE

Manufacturer Qualifications:

Not less than 25 years documented experience in manufacturing silicone roof coatings.

ISO 9001 certified.

Retain following Applicator Qualifications for Projects requiring a Product Plus or Labor and Material warranty.

Applicator Qualifications:

Approved by roof coating manufacturer.

Eligible to offer the specified manufacturer warranty.

Experienced with installation of the same or similar waterproofing materials.

Roof Coating:

UL Listed.

FM Class 1, 4470 approved.

NSF P151 Certified.

Adhesion Test: Conduct an adhesion test to all differing surfaces that are to receive coating. Test(s) shall be performed in accordance with manufacturer’s written instructions. Document results.

DELIVERY, STORAGE AND HANDLING

Deliver materials and products in their original and unopened packaging with legible labels showing the manufacturer’s name, lot numbers, and product identification visible.

Handling and Storage:

Store roof coating containers between 15 °F and 109 °F (-9 °C to 43 °C).

Store other materials in accordance with the manufacturer’s instructions.

Keep products out of direct sunlight.

Dispose of materials according to applicable regulations and laws.

PROJECT CONDITIONS

Ensure roof appurtenances are in place prior to surface preparation and roof coating application.

Weather Conditions: Proceed with the Work only when existing and forecasted weather conditions will permit materials to be installed in accordance with the manufacturer’s written instructions and requirements.

Substrate Conditions: Do not install materials over substrates that are damp, wet, or otherwise contaminated in such a way to prevent proper adhesion.

Apply roof coatings at temperatures above 0 °F (-18 °C) and below 120 °F (49 °C). Do not apply coating materials when temperatures are within 5 °F (3 °C) of the dew point.

Contact manufacturer for recommendations if applying coating to substrates over 120 °F (49 °C).

WARRANTIES

Select one of the following manufacturer warranties.

Manufacturer’s Product Warranty: Submit a written warranty from manufacturer agreeing to replace any defective product as defined by the Warranty, at no charge.

Warranty term: [10] [15] [20] years.

Product Plus Warranty requires applicator to be authorized by the manufacturer.

Manufacturer’s Product Plus Warranty: Submit a written warranty from manufacturer agreeing to replace any defective product as defined by the Warranty and pay the cost of labor to install the replaced product.

Warranty term: [10] [20] [30] years.

Labor and Material Warranty requires applicator to be authorized by the manufacturer and a Moisture Scan.

Manufacturer’s Labor and Material Warranty: Provide roof coating manufacturer’s labor and material warranty.

Warranty term: [10] [15] [20] years.

Installer’s Warranty: Submit roofing installer’s warranty, signed by Installer, covering the work of this section, including all components installed, for the following term:

Warranty term: [2] [5] [__] years.

PRODUCTS

MANUFACTURERS

Basis of Design: EndurisTM 3525 by Momentive Performance Materials
(www.siliconeforbuilding.com).

Substitutions: [Substitutions not permitted.] [Refer to Division 01 for Substitution Procedures.]

MATERIALS

Silicone Roof Coating:

Description: High solids, solvent-free, alkoxy-based, moisture-cured, silicone roof coating.

Physical Properties: Tested to ASTM D6694.

Tensile strength: 200 PSI, tested to ASTM D2370.

Elongation: 500 percent, tested to ASTM D2370.

Volume solids: 90 percent, tested to ASTM D2697.

ACCESSORIES

Seam Treatment and Flashing Materials:

EndurisTM Liquid Flashing by Momentive Performance Materials.

UltraSpanTM UST / USM pre-cured silicone transition sheets by Momentive Performance Materials.

Reinforcement Fabric: RF100TM series, 100 percent polyester spun-laced textile reinforcing fabric.

SWS Silicone SealantTM by Momentive Performance Materials.

Walkway Coating: EndurisTM 3525 protection silicone coating, [yellow,] [contrasting color,] with granules installed in coating at the minimum rate of 30 pounds per 100 square feet or until rejection.

Slip Sheet: TPO single ply membrane for use as a slip sheet under loose-laid materials and equipment such as sleepers, pipe supports, etc.

EXECUTION

EXAMINATION

Momentive has a Pre-Job Evaluation Checklist on their website to assist the construction team in evaluation of the existing roof for acceptability of EndurisTM product.

Conduct a roof inspection to determine the necessary repairs and the type(s) of materials to use. Manufacturer will not be responsible for any defects or deficiencies that will inhibit proper performance of the coating.

Preinstallation Testing: Conduct adhesion tests in accordance with manufacturer’s adhesion testing procedures to determine if primer or other surface preparation is required. Document adhesion test results.

Verification of Existing Conditions:

Walk the roof to identify defective items and issues that will affect the performance of the coating.

Examples of defective items are, but are not limited to: open seams, open laps, rusted panels, deficient sealant on storm collars, severe rust and corrosion, ponding conditions, poor condition of drain details and components, deteriorated sealant on surface-mounted counter flashings, deteriorated wall joint sealant, and pipe penetration flashing defects.

Inspect all roof drainage details for defects that will result in leaks or inhibit proper drainage. Notify Owner of any issues needing corrective action.

Identify existing roof leaks and notify Owner if any leaks are present that cannot be addressed as part of the Work.

Do not proceed with Work until deficiencies are corrected. Beginning of Work constitutes acceptance of substrate.

PREPARATION

Prior to performing any pressure washing, repair all existing roof leaks according to industry standards to prevent additional moisture intrusion.

Mask or otherwise protect surfaces not to be coated.

Review existing and imminent weather conditions including potential for extreme temperatures, relative humidity, frost, dew, and precipitation. Ensure that coating and accessory materials will have sufficient curing time.

Surface Preparation and Repairs:

Mechanically remove loose and flaking rust.

Replace or repair metal panels that are rusted through or nearly rusted through. Panel replacement shall be performed in accordance with industry standards.

Replace or repair severely damaged metal panels. Perform panel replacement in accordance with industry standards.

Seal all horizontal and vertical laps/seams with a minimum 3-inch width of 60 wet mils of liquid flashing.

Seal all seams of curbs, gaps, flashings, angle changes, and penetrations with minimum 60 wet mils of liquid flashing.

Inspect the roof ridges, hips, and eaves for the presence of foam closures and replace damaged or missing closures.

Inspect the roof for proper fastener placement at side and end laps. Install additional capped, gasketed fasteners, as needed, to comply with the following requirements:

Panel ends: Six (6) capped, gasketed fasteners installed per panel.

Intermediate fastener placement: Three (3) fasteners per panel.

Purlins (open framing systems): Install fasteners at side laps through each purlin support and spaced a maximum of 20 inches on center between the purlin supports.

Solid decking systems: Install fasteners at side laps at 20 inches on center.

Inspect the roof for loose or open side and end laps and install additional capped and gasketed fasteners, as needed, to close the laps.

Replace loose or missing fasteners using oversized, capped, gasketed fasteners.

Seal exposed fastener heads with liquid flashing.

Properly prepare and encapsulate the tops of all surface-mounted counter flashings with a minimum of 60 mils of liquid flashing. Perform adhesion tests as needed, and in accordance with the manufacturer’s instructions. Remove chalking from existing sealants.

Pitch Pans (aka Penetration Pockets): Pitch pans are not warrantable. The following are recommendations for sealing pitch pans:

Ensure the liquid flashing will adhere to the pitch pan filler by performing an adhesion test.

Cut a “V” at the base of the pitch pan penetration(s).

Apply a manufacturer-approved sealant to the base of the penetrations.

Encapsulate the entire pitch pan with a 3-course application of silicone coating and fabric reinforcement.

Rusted or corroded pitch pans should be removed and replaced with new pitch pans. The lines through them are to be cleaned and liquid flashing used as pitch pan filler.

Seal all horizontal sheet metal flashing joints such as copings and expansion joint covers, by using a bond breaker and then covering with a 2-inch-wide strip of 60 wet mils of liquid flashing or manufacturer-provided, compatible sealant. An example of a bond breaker is running a lumber crayon over the joint and then encapsulating it with the sealant.

Standing seam joints are excluded from this requirement.

Seal storm collars using liquid flashing or a compatible sealant approved by the coating manufacturer.

Pressure wash roof surface at 2500 to 4000 PSI utilizing manufacturer’s recommended roof preparation wash or approved biodegradable detergent; remove oils and other materials that could interfere with adhesion. Rinse with clean water until no bubbles or suds remain. Allow to dry thoroughly. Extra time for drying may be necessary if water is present at the laps or in ponding areas such as on the upslope side of curbs.

Double wash all areas where water collects as sediments and other debris will collect in these areas and can inhibit proper adhesion.

Seal exposed fasteners using liquid flashing.

Replace missing foam closure strips at eaves, ridges, and hips.

Required Condition of Surfaces: Clean, sound, dry, and free of materials, laitance, chalking, and loose coatings that could inhibit proper adhesion of coatings or sealants. Seal penetrations and flashings watertight prior to coating application.

APPLICATION

Apply roof coating in accordance with manufacturer’s instructions and approved Drawings.

Application rates will vary depending on the complexity of the project. The following rates do not consider waste. Applicator is required to calculate the waste factor needed. “Waste” includes material left in the buckets or barrels, on roller frame covers, inside hoses (when spray applied), etc. All application rates apply to both the roof and any walls or curbs to be coated.

Include the stretch out of the metal panels so that all ribs, flutes, and pans are properly coated.

Select one of the following application mil thicknesses based on 10, 15, or 20 year warranty requirements.

[Apply roof coating at rate of 1.5 gallons per 100 square feet to minimum 21 mils cured coating thickness.]

[Apply roof coating at rate of 2.0 gallons per 100 square feet to minimum 28 mils cured coating thickness.]

[Apply roof coating at rate of 2.5 gallons per 100 square feet to minimum 36 mils cured coating thickness.]

Apply coating by squeegee, brush, roller, airless sprayer, or a combination of these.

Apply liquid flashing by brush, trowel, gloved hand, or with a bulk caulking gun.

Final Roof Coating: Monolithic and seamless, encapsulating entire roof surface.

Walkway Protection is optional but strongly suggested when mechanical units or other equipment is located on the roof. Retain below if roof is subject to maintenance of rooftop equipment.

Walkway Protection: At roof access locations and around rooftop equipment requiring servicing. Apply walkway coating a minimum of 60 wet mils thick, simultaneously embedding roofing granules at the rate of approximately 35 pounds per 100 square feet (16 kilograms per 9.25 square meters). Walkway shall be a minimum of 30 inches (760mm) wide. Mask sides to achieve straight lines.

Lift and coat under all sleeper supports, including both wood and manufactured supports, for service lines and rooftop equipment such as mechanical units and satellite dishes. After coating has cured place a slip sheet under supports to protect the coating. Extend slip sheet past the edges of the sleeper a minimum of 1 inch on all sides.

CLEANING

Clean finished roof surface after completion; ensure that drainage components are not clogged.

PROTECTION

Protect roof coating from foot traffic and damage while curing. A cure time of up to 3 days is recommended to achieve full cure.

FIELD QUALITY CONTROL

Verify proper application rates by using a wet film gauge. Check wet film thickness every 100 to 200 square feet (9.25 to 18.5 square meters).

Verify cured mil thickness of coating at end of work and prior to warranty inspection.

Repair deficient areas with liquid flashing or roof coating, as applicable to size of deficient area.

Manufacturer’s project acceptance is subject to pre-job, progress, and final inspection by coating manufacturer or its designated third-party inspectors.

END OF SECTION