Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format.  It must be carefully reviewed and edited by the Architect or Acoustical Consultant to meet the requirements of the project.  Coordinate this section with other specifications sections and the Drawings.

Review and edit all items listed in blue font color or red font color.

Delete all Specifier Notes when editing this section by clicking on box outline and pressing “delete”.

SECTION 09000

Seamless Acoustic System

30 mm BASWA DTG Prime Frost Acoustic System

Specifier Note:  This section covers the BASWA DTG acoustic system. Consult BASWA acoustic North America, 855.902.2792 or info@baswana.com, for assistance in editing this section for your specific application.

PART 1 GENERAL

- SUMMARY

  - The BASWA DTG acoustic system is used to reduce reverberation time, making voice, music, and other sound much more intelligible.  Its design is based on a fine porous surface that appears to be solid, applied onto a mineral wool panel.  High frequency sound energy passes through the pores, into the mineral wool, and is converted into heat energy.  Low frequency sound energy vibrates the porous surface diaphragmatically, and is converted into heat energy.

  - The BASWA DTG acoustic system is a complete system. Components of the system are as follows: 26 mm (1”) BASWA DTG system panels, which consist of mineral wool with airtight foil backing, are mounted to a drywall grid framing assembly (in accordance with ASTM C636 and ASTM C754, or per local codes) at each framing member using a BASWA-approved ASTM C557 adhesive for metal and 1 5/8” phosphate-coated, fine-thread drywall screws and stainless steel discs. The panel surface is coated with hand-troweled BASWA Prime as a base layer and finished with a spray-applied Frost finish coat of BASWA Base. All components, including trims and tints, are supplied by the manufacturer.

  - The work described in this Section, as shown on Drawings, Finish Schedules, or as specified herein, shall be in accordance with the requirements of the Contract Documents.

- SECTION INCLUDES

  - Work in this Section includes all labor, materials, equipment, and services necessary to complete the BASWA DTG acoustic system, as shown on the drawings, finish schedules, and / or defined and specified herein.

RELATED SECTIONS

Specifier Note:  Edit the following list of related sections as required for the project.  List other sections with work directly related to this section.

  - Carpentry – See section 06200

  - Suspension Systems - See section 09226

  - Gypsum board – See section 09250

- QUALITY ASSURANCE

  - Certified installers

    - All Contractors shall be certified to install the BASWA DTG acoustic system by BASWA acoustic North America, LLC (855.902.2792 or info@baswana.com)

- SUBMITTALS

Samples /OR/ Mockup

Specifier Note:  Use i or ii as applicable to your project needs.

    - Samples: Provide two 8-1/2” x 11” samples of the BASWA DTG acoustic system in color as noted in Section 2.1 below. Samples must show the complete panel and all layers to be installed.

    - Job Site Mockup: Install a 4’ x 4’ mock-up of the BASWA DTG acoustic system replicating relative details and conditions.  Obtain mockup acceptance before any additional applications. Accomplish work to equal or exceed standard established by accepted job site mockup.

- TEST DATA

  - Acoustical performance data

    - Certified reports for Acoustical Performance Sound Absorption determined per ASTM C423 Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method for ‘Type E’ Mounting, conducted by an accredited, independent, testing agency, shall be submitted upon request and meet the following minimum requirements. Noise Reduction Coefficient (NRC) rating for the 30 mm (1 3/16”) system shall be 0.75 with frequency absorption coefficients as follows:

Frequency			Absorption

(Hz)				Coefficient

100				0.52

125				0.61

160				0.53

200				0.40

250				0.53

315				0.58

400				0.68

500				0.74

630				0.82

800				0.85

1000				0.82

1250				0.87

1600				0.95

2000				0.96

2500				0.97

3150				0.98

4000				1.00

5000				1.14

  - Fire test data

    - Certified reports for Surface Burning Characteristics determined per ASTM E 84 Twenty-Five Foot Tunnel Furnace Test Method, conducted by an accredited, independent, testing agency, shall be submitted upon request and meet the following minimum requirements:

            - Class A Flame Spread Classification

              - 20 Flame Spread or less

        - 15 Smoke Development or less

  - Mold test data

    - Certified reports for Resistance to Growth of Mold on the Surface of Interior Coatings in an Environmental Chamber determined per ASTM D3273-16, conducted by an accredited, independent, testing agency, shall be submitted upon request and meet the following minimum requirements:

            - 10 / 10 – Week 4 – No Defacement

  - Light reflectance test data

    - Certified Reports for Luminous Reflectance Factor of Acoustical Materials by Use of Integrating Sphere Reflectometer determined by ASTM E 1477-98, conducted by an accredited, independent, testing agency, shall be submitted upon request and meet the following minimum requirements:

            - Light Reflectance Value ‘L’ a minimum of 89.6.

  - VOC emission test data

    - For each system component, Certified Reports for California Department of Public Health CDPH / EHLB / Standard Method Version 1.2, 2017, conducted by an accredited, independent testing agency, shall be submitted upon request and meet the following minimum requirements:

            - Classroom – VOCs of Concern: ≤½ Chronic REL, Compliant; Formaldehyde: ≤9.0 μg/m3, Compliant; TVOC range: ≤ 0.5 mg/m3.

            - Office – VOCs of Concern: ≤½ Chronic REL, Compliant; Formaldehyde: ≤9.0 μg/m3, Compliant; TVOC range: ≤ 0.5 mg/m3.

  - Thermal performance

            - W/(m*K) of 0.04 or greater.

  - Adhesive strength test data

    - Certified Reports for Tensile Properties of Adhesive Bonds determined by ASTM D897, conducted by an accredited, independent, testing agency, shall be submitted upon request and meet the following minimum requirements:

            - Average Tensile Strength of 106 (psf) with an avg Maximum Force applied of 431 (lbf)

SUSTAINABILITY DATA

Specifier Note:  List reference standards in this section, complete with designations and titles. Include or delete Section(s) below as applicable to your project needs.

  - LEED v.5

    - Energy & Atmosphere

            - Enhanced Energy Efficiency

              - Provides a high R-Value

          - Materials & Resources

            - Low-Emitting Materials

              - Pass CA Dept. of Public Health CDPH Standard Method Vers 1.2, 2017.

              - No formaldehyde.

            - Building Product Selection and Procurement

              - System’s total recycled content by weight is 83.4%; 52.2% pre-consumer and 31.2% post-consumer.

              - Detailed information on the components of panels and finishes are third-party tested and documented.

              - HPD reported and current for each system component.

            - Construction & Demolition Waste Management

              - Typical installation results in less than 3% waste. Panel scraps may be acceptable in mixed-glass recycling facilities. Plastic buckets containing wet materials are reusable and recyclable. Unused panels and buckets can be saved for future projects.

          - Indoor Environmental Quality (EQ)

            - Enhanced Air Quality

              - Pass CA Dept. of Public Health CDPH Standard Method Vers 1.2, 2017. VOCs of Concern: ≤½ Chronic REL, Compliant; Formaldehyde: ≤9.0 μg/m3, Compliant; TVOC range: ≤ 0.5 mg/m3.

              - No formaldehyde.

            - Occupant Experience

              - NRC of 0.75 per ASTM C423 Type E Mounting.

              - Light reflectance L value of 91.03 per ASTM E 1477-98.

              - Provides high R-Value.

              - BASWA systems can be intercoupled to provide flexible sound absorption between zones while maintaining seamless aesthetics.

              - BASWA systems can be installed in covered, outdoor areas to mitigate sound transference to indoor spaces.

            - Accessibility and Inclusion

              - NRC of 0.75 per ASTM C423 Type E Mounting.

  - HPD reported and current

- REFERENCES

Specifier Note:  List reference standards in this section, complete with designations and titles. Include or delete Section(s) below as applicable to your project needs.

  - ASTM C 423-23:  Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method. ‘Type E’ Mounting.

  - ASTM E 795-23:  Standard Practices for Mounting Test Specimens During Sound Absorption Tests.

  - ASTM E 84:  Standard Test Method for Surface Burning Characteristics of Building Materials

  - ASTM E 1477-98: Standard Test Method for Luminous Reflectance Factor of Acoustical Materials by Use of Integrating Sphere Reflectometer.

  - ASTM D 3273-21: Standard Test Method for Resistance to Growth of Mold on the Surface of Interior Coatings in an Environmental Chamber

  - ASTM D897: Standard Test Method for Tensile Properties of Adhesive Bonds

  - ASTM C557: Standard Specification for Adhesives for Fastening Gypsum Wallboard to Wood Framing

  - ASTM C636: Standard Practice for Installation of Metal Ceiling Suspension Systems for Acoustical Tile and Lay-In Panels

  - ASTM A641/A641M: Standard Specification for Zinc–Coated (Galvanized) Carbon Steel Wire

  - ASTM C635/C635M: Standard Specification for Manufacture, Performance, and Testing of Metal Suspension Systems for Acoustical Tile and Lay-in Panel Ceilings

  - ASTM C645: Standard Specification for Nonstructural Steel Framing Members

  - ASTM C754: Standard Specification for Installation of Steel Framing Members to Receive Screw-Attached Gypsum Panel Products

  - ASTM A653/A653M: Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process

- DELIVERY, STORAGE, AND HANDLING

  - Ship and deliver in protective packaging to prevent freight damage.

  - Allow materials to become acclimated to Project conditions before installation.

  - Store materials in accordance with manufacturer's recommendations in a fully enclosed space where materials will be protected against damage from moisture, surface contamination, and other causes.

  - Protect BASWA Prime and Base from freezing.  Product that has frozen cannot be used and is not warranted.

- PROJECT CONDITIONS

  - Environmental Requirements:  Comply with requirements of referenced plaster application standards and recommendations of product manufacturer for environmental conditions before, during, and after installation.

  - Temperature Requirements:  Where ambient outdoor temperature at the building site is less than 55⁰ F (13⁰ C), a temperature of not less than 55⁰ F and not more than 80⁰ F shall be maintained continuously in the area of the installation for a period of not less than one week prior to the application of BASWA DTG acoustic system. Temporary heat shall be evenly distributed to prevent concentrated uneven heat or cold on the BASWA DTG acoustic system or its substrate. Acceptable temperature range shall be maintained until the permanent HVAC system is activated.

  - Ventilation:  Ventilate building spaces as required to remove excess moisture to promote drying of the applied materials.

  - Prevent strong fluctuations in temperature and relative humidity, care must be taken to ensure that the dew point is not located on the surface or within the BASWA acoustic system. Moisture caused by condensation in the room air can cause damage to the BASWA acoustic system.

  - Protect contiguous work from soiling, splattering, moisture deterioration, and other harmful effects that may be caused by the application of the materials.

PART 2 – PRODUCTS

      - MATERIALS

        - Materials and installation shall be based upon the BASWA DTG acoustic system’s performance, specifications, planning documents, and details as supplied by BASWA acoustic North America, LLC, www.baswana.com, 855.902.2792, info@baswana.com.  No substitutions.

        - The BASWA DTG acoustic system shall be provided in a total system thickness of 30 mm.

        - The BASWA DTG acoustic system shall consist of a 26 mm (1”) BASWA DTG system panel, BASWA Prime, and a spray-applied Frost finish coat of BASWA Base.

        - Trim

          - All corner beads, terminations, control joints, or other trim pieces shall be white vinyl BASWA proprietary beads.  Specialty beads and reveals in profiles approved by BASWA acoustic shall be white vinyl beads manufactured by either Trim Tex or Vinyl Corp.

          - Trims shall be installed with Trim-Tex 847 Spray Adhesive; no other adhesive is approved.

Specifier Note: Use i for the standard natural white applications or ii for custom tinted applications, as applicable to your project needs.

Color

    - The spray-applied Frost finish coat of BASWA Base shall be provided in BASWA standard natural white.

    - The spray-applied Frost finish coat of BASWA Base shall be tinted by the addition of BASWA Colors tint additive. Color shall be selected by the Architect or as noted in the Finish Schedule.

Specifier Note:  Determine if the area of the application will be exposed to unusually abusive soiling conditions. Include or delete section f as applicable to your project needs.

BASWA Protect surface protectant

    - A spray application of BASWA Protect shall be applied to the finished surface after system has completely dried for a minimum of 12 hours. The non-bridging coating resists stains and improves cleanability without negatively affecting acoustical absorption.  Application and coverage shall be per manufacturer’s approved application guidelines.

PART 3 – EXECUTION

      - INSPECTION

        - Examine areas where, and conditions under which, the BASWA DTG acoustic system is to be installed.  Correct any conditions detrimental to the proper and timely completion of the work.   Do not proceed until unsatisfactory conditions have been corrected to permit the proper installation of the work.

        - Verify that all mechanical and electrical services within the area of application have been roughed in at the appropriate depth relative to the thickness of the system; tested and approved, prior to commencement of application.  Review approved details provided by BASWA acoustic for verification.

      - ACCEPTABLE SUBSTRATE

        - General

          - The BASWA DTG acoustic system must be installed over a framing system that is in accordance with ASTM C636 and ASTM C754, or per local codes.

          - Fasteners used to attach panels to framing should be 1 5/8” phosphate-coated, fine thread, bugle head drywall screws.

          - All substrates for the application shall not vary from plumb by more than a total of 1/8” in 10’.

Framing assembly

Specifier Note:  Determine the substrate condition(s) in the area(s) where BASWA is to be installed.  Include and / or delete Section(s) below as applicable to your project needs.

          - Recycled content of steel products: post-consumer recycled content plus one-half of pre-consumer recycled content not less than 25%.

          - Framing members, general: comply with ASTM C754 for conditions indicated.

            - Protective coating: ASTM A 653/A 653M, G40, hot dip galvanized unless otherwise indicated.

          - Drywall grid assembly

            - Complies with applicable requirements per ASTM C645, direct hung system composed of drywall grid main beams and cross-furring drywall suspension tees that interlock.

            - Framing system

              - Deflection criteria: L/360 per ASTM C635 standard practice for metal suspension systems.

              - Galvanized steel: G40 double-web tee, hot dipped galvanized steel.

            - Framing members

              - Drywall grid main beams: 6’ on center

              - 6’ Cross tees: 2’ on center.

              - 2’ Cross tees: 1’ on center (to create a 2’ x 1’ grid pattern).

              - BASWA DTG panels are 2’ x 4’ and framing should reflect this size.

              - The framing cannot have any residue or oil finish.  If it does the framing must be wiped down with acetone.

            - Attachment devices: size for five times the design load indicated in ASTM C635/C635M, Table 1, “Direct Hung” unless otherwise indicated.  Comply with seismic design requirements, if applicable.

            - Wire hangers, braces, and ties: provide wires complying with the following requirements:

              - Zinc-Coated, Carbon-steel wire: ASTM A 641/A641M, Class 1 zinc coating, soft temper.

              - Size: Minimum 12 gage per ASTM C636

      - INSTALLATION

        - General Information and Requirements

          - Hand Tools

            - All application of BASWA DTG system panels, BASWA Prime, and spray-applied Frost finish coat of BASWA Base must be facilitated by using the proper BASWA stainless steel flat or notched hand trowels available from BASWA acoustic.  The proper notched gauging trowels and smoothing trowels shall be used at each step noted below to control material thickness.

          - BASWA Jet Pump (Required for application of Frost Finish Coat)

            - The spray-applied Frost finish coat of BASWA Base must be applied using the BASWA Jet Pump. This can be purchased through BASWA acoustic.

          - Lighting

            - Inherent in all hand troweled product applications, minor acceptable trowel marks or imperfections in the finished surface may occur and become exposed or exaggerated under critical lighting.  Ensure that the lighting used during the entire installation process replicates the actual finished lighting.  All skylights, clearstory windows and other openings which naturally light the area of the installation shall be uncovered during the entire installation to represent finished conditions.

          - Drying Times

            - Drying time for the BASWA Prime is typically 24-48 hours, however, drying times may be longer due to unusual on-site conditions.  Prior to proceeding with any additional work, ensure BASWA Prime Coat is completely and thoroughly dry.

          - Finished Coat Installation

            - Application of the Frost finish coat of BASWA Base shall be facilitated in one operation at each area of installation.

          - Staging

            - Generally, the BASWA DTG acoustic system can be installed using rolling towers or mobile scaffolds.  A full dance floor scaffolding is NOT necessary.

          - Access Doors in BASWA

            - BASWA proprietary access doors are GFRG and include a mud bump to provide a finishing edge for the BASWA Base material.  BASWA proprietary access panels measure 24” x 24”.

            - Access Doors used in the BASWA DTG acoustic system can be flanged or trimless. Trimless access door model number AT5020, sized as required, manufactured by Acudor Products Inc. (800.722.0501) or similar.  Install per detail approved by BASWA acoustic.

          - Securing to BASWA

            - Light fixtures, ornamentation, speakers, cover plates, or any other items cannot be attached to the BASWA DTG acoustic system. Ensure items are secured to a BASWA Installation Platform, proper blocking, or other approved attachment system independent of the BASWA DTG acoustic panel per BASWA-approved details.

        - Installation procedures

          - Install BASWA DTG acoustic system in accordance with BASWA acoustic’s planning documents, installation instructions, and details. Installation shall start only after all other work in the area of the installation has been completed.

            - BASWA DTG system panels

              - Adhere FRK face of BASWA DTG system panels to each framing member using a BASWA-approved ASTM C557 adhesive for metal, ensuring contact with the grid. Once adhered, install screws in accordance with BASWA planning documents.

              - Abut subsequent panels flush and joint-tight. Stagger joints between panels.

            - Trim

              - Install white vinyl BASWA proprietary trim pieces. Trims shall be installed with Trim-Tex 847 Spray Adhesive; no other adhesive is approved.

            - Prime Trim

              - 1.	While installing the DTG System panels, apply BASWA Prime to cover the surface of the trim. When the Prime coat is dry sand smooth.

            - BASWA Prime Coat

              - Prior to applying the base-layer coat of BASWA Prime, ensure materials are thoroughly mixed.  Hand-deliver and gauge a 2.0 mm thick layer and trowel smooth.

              - Sand smooth when completely dry.

              - Do not tint BASWA Prime

Specifier Note:  Determine if BASWA is specified in standard natural white or a custom color. Remove section 5 or 6 accordingly, as applicable to your project needs.

            - Spray-applied Frost Finish Coat of BASWA Base (Standard Natural White)

              - Prior to applying the Frost finish of BASWA Base, ensure product is thoroughly mixed and several pails are continuously batched together to provide even, consistent color.

              - Using a BASWA Jet Pump in 2-3 spray applications, deliver a 1.0-2.0 mm thick layer of BASWA Base, achieving a quality level consistent with accepted samples or mock-up.

              - Note the BASWA systems are hand-crafted finish products. Inherent in all hand-crafted products, minor acceptable trowel marks and other imperfections in the finished surface may occur which are only visible at certain times of day or under certain critical lighting conditions.  The finish should be critically viewed only under end-use lighting conditions

            - Spray-applied Frost Finish Coat of BASWA Base in Number / Name of Color (Custom Color)

              - Color is specified. Prior to applying the Frost finish coat of BASWA Base, add one vial of BASWA Colors tint additive per bucket.  Ensure product is thoroughly mixed and several buckets are continuously batched together to provide even, consistent color.

              - Using a BASWA Jet Pump in 2-3 spray applications, deliver a 1.0 – 2.0 mm thick layer of BASWA Base, achieving a quality level consistent with accepted samples or mock-up.

              - Note that BASWA systems are hand-crafted finish products. Inherent in all hand-crafted products, minor acceptable trowel marks and other imperfections in the finished surface may occur which are only visible at certain times of day or under certain critical lighting conditions. The finish should be critically viewed only under end-use lighting conditions.

END OF SECTION