DPP content proposal presented during the Ecodesign Forum Expert Group Meeting

19th of March 2026

Product identification and classification

| Data Requirement | Reference Methodology |
| --- | --- |
| Unique Product ID | Serialised Global Trade Item Number (SGTIN) |
| Batch ID | GTIN + Lot number |
| Model ID | GTIN - 13 |
| Product ESPR Category | ESPR classification (knitted, woven, denim product) |
| Product PEFCR Category | PEFCR for Apparel and Footwear |
| Commodity Codes: HS and TARIC | WCO Harmonized System (HS) TARIC XML Schema |

Producer identification

| Data Requirement | Reference Methodology |
| --- | --- |
| Manufacturer(*) unique operator identifier | Party Global Location Number (Party GLN) and Economic Operators Registration and Identification (EORI), if available |
| Manufacturer(*) name | Free text ISO/IEC 6523 |
| Manufacturer(*) postal address | Free text ISO/IEC 6523 |
| Manufacturer(*) contact information | [email or other electronic means] |
| Unique facility identifier(s) | Global Location Number (GLN) |

* (and, if relevant, importer and other responsible operator)

Product information

| Data Category | Data Field | Reference Methodology |
| --- | --- | --- |
| Product information - Material | Fiber composition | Textiles Labelling Regulation (TLR)(Clean dry mass) |
|  | Components specification | GTIN-13 |
| Product Information – Mechanical properties | Robustness score | Methodology in the ESPR prep study |
|  | Visual inspection | Test results, ISO 15487 |
|  | Spirality | Test results, ISO 16322-3 (%) |
|  | Dimensional change | Test results, ISO 3759 (%) |
|  | Conformity certification | Verification rules specified in ESPR DA of textiles |
| Product information – Chemical properties | Substances of Concern (SoC) | IUPAC / CAS / EC |
|  | Location of the SoC | Free text |
|  | Concentration of SoC | % w/w |
|  | Relevant instructions for the safe use | Free text |
|  | Information relevant for disassembly, reuse, recycling and management at EoL | Free text |
| Product information - recyclability | Recyclability score | Methodology in the ESPR prep study |
|  | Conformity certification | Verification rules specified in ESPR DA of textiles |
| Product Information – Recycled content | Recycled content (%) | % w/w (ESPR methodology based on ISO 14021:2016) |
|  | Recycled content type | Post-industrial, pre- and post-consumer waste |
|  | Weight (excluding trims) | [kg] EN ISO 80000-1 (Quantities and units) TLR (clean dry mass) |
|  | Amount of recycled material | [kg] Methodology to be proposed in detail under ESPR DA of textiles (based on ISO 14021:2016) |
|  | Conformity certification | Verification rules specified in ESPR DA of textiles |
| Product Information – Other EU Law | Organic content (%)* | % w/w (Reg (EU) 2018/848) |
|  | Amount of organic material* | [kg] Methodology to be proposed in detail under ESPR DA of textiles (in compliance with Organic Regulation (EU) 2018/848) |
|  | Conformity certification* | Verification rules specified in ESPR DA of textiles |
|  | EU Ecolabel | EU Ecolabel |
| Product Information – Product footprint | Product carbon/environmental footprint – Compared to benchmark | [%] Methodology to be proposed in detail under ESPR DA on textiles based on PEFCR (Climate change / Single score) |
|  | Product carbon/environmental footprint – Absolute value | [kg CO2e (or env. points)/kg] Methodology to be proposed in detail under ESPR DA of textiles based on PEFCR (Climate change / Single score) |
|  | Weight | [kg] EN ISO 80000-1 (Quantities and units). It needs to align with the methodology of the PEFCR |
|  | Conformity declaration | Verification rules specified in ESPR DA of textiles |
| Product Information – Manuals and instructions | Care instructions | TLR / ISO 21600:2019 |
|  | Repair instructions * | Free text |
|  | Contact of repair services offered by brand * | Free text ISO/IEC 6523 |
|  | Warranty duration | ISO 22059:2020 / Consumer Rights Directive as amended by Empowering Consumers Directive |

* Voluntary field