(Add Mine Contact & Address, including EMAIL)

Attention: (add Contact name)

Date:

Installation Details (add)

Notification - Completion of Installation of Safescape

Xxxxx (Site Name) wish to notify Safescape that the Safescape Laddertube in the               level to the                   is now complete. Xxxxx (Installers details including Certificate #) installed Safescape Laddertube as per scope of work and to required standards, inclusive of the Static Line fall arrest system.

Site Installers:

Names and signatures

Once the Safescape is complete and this document signed by the client and or installation team, please return via email to Safescape at your earliest convenience.  All installation paperwork to be provided including JSA, Installation Plan and Checklists to be included.

Any issues with the Safescape modules or consumables to be noted - YES/NO

If yes, please note:

Other Comments:

| Fall Arrest Vi-Go Shuttle – As noted in AS/NZS 1891.3 2009 – 1 Fall-arrest devices complying with EN 353-1, EN 353-2 or EN 360 are acceptable for use in Australia and New Zealand. Fall arrest cable termination point at upper anchor location: Client has been advised that the supplier certification of the termination point (swaging) is subject to quarterly visual inspections to ensure there are no visual defects. | Yes | No |
| --- | --- | --- |
| - Shuttle dimension must be adhered to. The shuttle may only be used if the dimension is in the range of 116.2mm +/- 2mm1 |  |  |
| - The supplied fall arrester has been checked for perfect functioning |  |  |
| - The gap of the fall arrester between the slider and shuttle housing must not be more then 6mm |  |  |
| - The upper cable anchor point is always above the person to be secured |  |  |
| - Only anti-corrosive screw connections have been used |  |  |
| - The cable is adequately pre-tensioned |  |  |
| - A trial run has been carried out |  |  |
| - The user guide has been received by the operating company – this guide outlines maintenance requirements and schedule for the system |  |  |
| - Soll Vi-Go shuttles received by site representative |  |  |

1Note the FID is measured from the top of the rope runner edge laterally to swivel attachment point, covering the fall indication window

Thank you for installing a Safescape. Please note that photographs were taken at your site as a part of the installation process and may be used on Safescape marketing and promotional materials. As a Safescape client, you may also be updated via email from time to time with industry developments & product innovation news. If you do not wish to receive future communication from Safescape, please tick here.

Client Representative          ______________________________     _________________

Signed			                        Dated