Drinking Water Quality and Compliance

Town of Gull Lake – Annual Notice to Consumers






2025
Introduction

The Water Security Agency and the Ministry of Environment requires that at least once each year waterworks owners provide notification to consumers of the quality of water produced and supplied as well as information on the performance of the waterworks in submitting samples as required by a Minister’s Order or Permit to Operate a waterworks.  The following is a summary of the (community name) water quality and sample submission compliance record for the (inset applicable time period here) time period.  This report was completed on (insert date here) (must be completed before June 30 each year on a calendar year based reporting frequency).  Readers should refer to Water Security Agency’s Municipal Drinking Water Quality Monitoring Guidelines, June 2015, EPB 502 for more information on minimum sample submission requirements and the meaning of type of sample.  Permit requirements for a specific waterworks may require more sampling than outlined in the department’s monitoring guidelines.  If consumers need more information on the nature and significance of specific water tests, for example, “what is the significance of Selenium in a water supply”, more detailed information is available from:  http://www.hc-sc.gc.ca/ewh-semt/pubs/water-eau/index_e.html 
.

Water Quality Standards 

Bacteriological Quality



Regular Samples
Regular Samples
  # of Positive Regular

Parameter/Location
Limit


Required

Submitted
  Submitted (%)

Total Coliform 

0 Organisms/100 mL
      53

         53                         0
E. coli


0 Organisms/100 mL
______________
_______________  __________________

Background Bacteria
Less than 200/100 mL
______________
_______________  __________________
Water Disinfection – 

Chlorine Residual in Distribution System for Test Results Submitted with Bacteriological Samples


Minimum
Total Chlorine
Free Chlorine
# Tests
# Tests
# Adequate

Parameter
Limit
Residual Range
Residual Range
Required
Submitted
Chlorine (%)
Chlorine
0.1 mg/L free OR


Residual
0.5 mg/L total                0.50 – 1.86
     0.04 – 1.32
     53
     53

    100
Water Disinfection - Free Chlorine Residual for Water Entering Distribution System from Waterworks Records-From Water Treatment Plant Records



Test Level
# Tests                # Tests Not Meeting

Parameter
Limit (mg/L)
Range
Performed          Requirements

Free Chlorine Residual
at least 0.20
0.22 – 2.12
    365                          0

A minimum of 0.1 milligrams per litre (mg/L) free chlorine residual is required for water entering the distribution system.  Tests are normally performed on a daily basis by the waterworks operator and are to be recorded in operation records.  This data includes the number of free chlorine residual tests performed, the overall range of free chlorine residual (highest and lowest recorded values) and the number of tests and percentage of results not meeting the minimum requirement of 0.1 mg/L free chlorine residual.

Turbidity – From Water Treatment Plant Records

Parameter
Limit
Test Level
# Tests Not Meeting
Maximum
# Tests           # Tests

# Tests


(NTU) 
Range
Requirements
Turbidity (NTU)
Required        Performed
Performed

Turbidity
   1.00             0.01-0.25
        0
      0.25
       365                365
____________

Chemical – Health Category 

All waterworks serving less than 5000 persons are required to submit water samples for SE’s Chemical Health category once every 2 years.  The Chemical Health category includes analysis for arsenic, barium, boron, cadmium, chromium, fluoride, lead, nitrate, selenium and uranium.

The last sample for Chemical Health analysis was submitted on (insert date).  Sample results indicated that the provincial drinking water quality standards were not exceeded.  (Use this one if review indicates that there were no exceedences).  (OR)  Samples exceeded provincial water quality standards for the following parameters:  (Use only the applicable portions of table below for which values have been exceeded).

Limit
Limit
Sample
# Samples

Parameter
MAC(mg/L)
IMAC (mg/L)
Result(s)
Exceeding Limit

Arsenic
0.010

    4.2
________________   * Results expressed

Barium
1.0

    5.7
________________ 
  as average values 

Boron

5.0
    0.5
________________      for communities or
Bromate
0.01

_________
________________      waterworks that

Cadmium
0.005

    <0.15
________________      fluoridate drinking
Chlorate
1.0

_________
________________      water supplies or

Chlorite
1.0

_________
________________      those with elevated

Chromium
0.05

   <0.19 
________________      concentrations of
Fluoride (avg*)
1.5

_________
________________      fluoride or nitrates.
Lead
0.01

   <0.07
________________      
Nitrate (avg.*)
45.0

_________
________________      
Seleniuum
0.01

   <1.13
________________
Uranium
002

    0.70
Chemical – Trihalomethanes (THMs)and Haloacetic Acids (HAAs)
Parameter
THMs

Sample
# Samples
# Samples

Limit (mg/L)

Result (average) 
Required
Submitted

Trihalomethanes
0.1
     133                4 (1 every 3 months)
        4
Haloacetic Acids
    0.08

        0.270  
  4 (1 every 3 months)
               4
Note:  Only water supplies derived from surface water or groundwater under the influence of surface water are required to monitor for THMs and HAAs.  Waterworks using groundwater sources beyond the influence of surface water do not need to report THMs or HAAs since sampling/analysis will not likely have been performed unless otherwise noted in the waterworks permit to operate
General Chemical 



Aesthetic 

Sample Results
# Samples # Samples


Parameter
Objectives * (mg/L)
(average)
Required     Submitted

Alkalinity
500
589
1

1
Bicarbonate
No Objective
719
1
1
Calcium
No Objective
85
1
1

Carbonate
No Objective
0

1
1
 

Chloride
250
30.6
1
1      

Conductivity
No Objective
1990
1
1


Hardness
800
603
1

1
Magnesium
200
95
1

1
PH
No Objective
7.9

1
1
Sodium
300
309

1
1
Sulphate
500
542.8

1
1
Total dissolved     


Solids                   1500
                         1798                     1
                  1
All waterworks serving less than 5000 persons are required to submit water samples for SE’s General Chemical category once every two years if a ground water source and once per three months every second year if a surface water or blended surface/groundwater source.  The General Chemical category includes analysis for alkalinity, bicarbonate, calcium, carbonate, chloride, conductivity, hardness (as CaCO3), magnesium, sodium, sulphate and total dissolved solids.

The last sample for General Chemical analysis was required on (insert year required) and submitted on (insert date) (use this statement if a groundwater supply).  The last sets of quarterly samples for General Chemical analysis were required on (insert year or sample submission period required) and were submitted on (insert dates) (use this statement if a surface source or blended source).  Sample results indicated that there were no exceedences of the provincial aesthetic objectives for the General Chemical category (use this statement if there were no exceedences). (OR)  Samples exceeded provincial aesthetic objectives for the General Chemical category for the following parameters: (use only the applicable portions of the table below for which values have been exceeded).
*Objectives apply to certain characteristics of or substances found in water for human consumptive or hygienic use.  The presence of these substances will affect the acceptance of water by consumers and/or interfere with the practice of supplying good quality water.  Compliance with drinking water aesthetic objectives is not mandatory as these objectives are in the range where they do not constitute a health hazards.  The aesthetic objectives for several parameters (including hardness as CaCO3, magnesium, sodium and total dissolved solids) consider regional differences in drinking water sources and quality.

More information on water quality and sample submission performance may be obtained from:


Town of Gull Lake 

Emella Waiser - Administrator


P.O. Box 150 Gull Lake, SK 

306-672-3361 Fax: 306-672-3777

gulllaketown.admin@sasktel.net
[image: image1.jpg]Saskatchewan
Ministry of
Environment




                                                                          [image: image2.jpg]Water Securtty
Agency

O



                   

[image: image1.jpg][image: image2.jpg]