Document Title: ASD FieldSpec 4 Technical Specifications

Rev Date: Sept 2025

| Layout Preview | English Text | Translated Text |
| --- | --- | --- |
|  | A SciAps Company |  |
|  | Analyze Your World |  |
|  | Product |  |
|  | FieldSpec 4 Hi-Res NG |  |
|  | FieldSpec 4 Hi-Res |  |
|  | FieldSpec 4 Standard-Res |  |
|  | FieldSpec 4 Wide-Res |  |
|  | Spectral range |  |
|  | 350-2500 nm |  |
|  | 350- 2500 nm |  |
|  | 350- 2500 nm |  |
|  | 350-2500 nm |  |
|  | Resolution |  |
|  | 3 nm @700 nm 6 nm@ 1400/2100 nm |  |
|  | 3 nm @700 nm 8 nm@ 1400/2100 nm |  |
|  | 3 nm@700 nm 10 nm@ 1400/2100 nm |  |
|  | 3 nm @700 nm 30 nm@ 1400/2100 nm |  |
|  | Noise Equivalent Radiance (NEdL) |  |
|  | VNIR SWIR 1 SWIR2 |  |
|  | 1.0 x 1o-9 W/cm2/nm/sr @ 700 nm 8.0 x 10-9 W/cm2/nm/sr@1400 nm 8.0 x 10-9 W/cm2/nm/sr@2100 nm |  |
|  | 1.0 x 1o-9 W/cm2/nm/sr @ 700 nm 1.4 x 1o-9 W/cm2/nm/sr @ 1400 nm 2.2 x 10-9 W/cm2/nm/sr@ 2100 nm |  |
|  | 1.0 x 10-9 W/cm2/nm/sr@700 nm 1.2 x 1o-9 W/cm2/nm/sr @ 1400 nm 1.9 x 10-9 W/cm2/nm/sr@2100 nm |  |
|  | 1.0 x 1o-9 W/cm2/nm/sr @ 700 nm 1.5 x 1o-9 W/cm2/nm/sr @ 1400 nm 2.8 x 10-9 W/cm2/nm/sr@2100 nm |  |
|  | ASD FieldSpec 4 Instrument Details |  |
|  | Spectral sampling (bandwidth) |  |
|  | 1.4 nm @350-1000 nm, 1.1 nm @1001-2500 nm |  |
|  | Scanning time |  |
|  | 100 milliseconds |  |
|  | Stray light |  |
|  | VNIR 0.02%, SWIR 1 & 2 0.01% |  |
|  | Wavelength reproducibility |  |
|  | 0.1 nm |  |
|  | Wavelength accuracy |  |
|  | 0.5 nm |  |
|  | Maximum radiance |  |
|  | VNIR 2X solar, SWIR 10X solar |  |
|  | Channels |  |
|  | 2151 |  |
|  | Detectors |  |
|  | VNIR detector (350-1000 nm) 512 element silicon array |  |
|  | SWIR 1 detector (1001-1800 nm) graded index lnGaAs photodiode, two stage TE cooled |  |
|  | SWIR 2 detector (1801-2500 nm) graded index lnGaAs photodiode, two stage TE cooled |  |
|  | Input |  |
|  | 1.5 m fiber optic (25° field of view). Optional narrower field of view fiber optics available. |  |
|  | Weight |  |
|  | 5.44 kg (12 lbs) |  |
|  | Calibration |  |
|  | Wavelength, reflectance, radiance*, irradiance*. All calibrations are NIST traceable (*radiometric calibrations are optional). |  |
|  | Computer |  |
|  | Windows® 7 64-bit laptop (instrument controller) |  |
|  | Warranty |  |
|  | One year full warranty including expert customer support |  |
|  | For service of ASD products, contact |  |
|  | ASD now part of the SciAps family of portable analyzers. |  |