Document Title: FieldSpec 4 vs reveNIR Comparison Chart

Rev Date: Dec 2025

| Layout Preview | English Text | Translated Text |
| --- | --- | --- |
|  | FieldSpec4 vs reveNIR Comparison Chart |  |
|  | Feature |  |
|  | FieldSpec 4 |  |
|  | ReveNIR |  |
|  | Best Choice For |  |
|  | Maximum spectral quality and research-grade measurements |  |
|  | Fast, portable field analysis and mineral identification |  |
|  | Use Style |  |
|  | Backpack or benchtop |  |
|  | Handheld and lightweight |  |
|  | Spectral Power |  |
|  | Exceptional — high resolution, excellent signal-to-noise |  |
|  | Excellent — optimized for field use |  |
|  | Measurement Type |  |
|  | Non-contact |  |
|  | Contact |  |
|  | Special Capabilities |  |
|  | Radiance & Irradiance measurements |  |
|  | Mineral Matching with built-in libraries |  |
|  | Ease of Use |  |
|  | Advanced, requires laptop |  |
|  | Simple, app-based operation |  |
|  | Field Ready |  |
|  | Portable, but not handheld |  |
|  | Fully handheld with hotswap batteries |  |
|  | Connectivity |  |
|  | Offline |  |
|  | Cloud-connected with GPS and cameras |  |
|  | Ideal For |  |
|  | Remote sensing, lab work, research |  |
|  | Mining, agriculture, on-site analysis |  |
|  | Delivery |  |
|  | 4–5 weeks after order |  |
|  | Typically faster |  |
|  | Real-World Examples |  |
|  | Used for NDVI, water studies, remote sensing |  |
|  | Used for mineral matching in exploration |  |
| [BACK] |  |  |
|  | Wavelength Range |  |
|  | Spectral Range: 350-2500 nm |  |
|  | Spectral Range: 350-2500 nm |  |
|  | Spectral Resolution (Standard-Res) |  |
|  | 3nm @ 700 nm 10 nm @ 1400 nm 10 nm @2100 nm |  |
|  | VNIR: 1.5 nm spectral average SWIR1 and SWIR2: ≤8 nm spectral average |  |
|  | Wavelength Accuracy |  |
|  | ±0.5 nm (average error of wavelength calibration fit; ±1.0 nm for any one line) |  |
|  | VNIR: 0.5 nm SWIR 1: 2.7 nm SWIR 2: 2.7 nm |  |
|  | VNIR detector |  |
|  | 512 element silicon array (350- 1000 nm) |  |
|  | CCD array – 2500 pixels (350-975 nm) |  |
|  | SWIR1 detector |  |
|  | InGaAs photodiode, TE cooled (1001-1800 nm) |  |
|  | Standard InGaAs – 256 pixels (976-1690 nm) |  |
|  | SWIR2 detector |  |
|  | InGaAs photodiode, TE cooled (1801-2500 nm) |  |
|  | Extended InGaAs – 256 pixels (1691 -2500 nm) |  |
|  | Signal to Noise Ratio |  |
|  | VNIR: 13000:1 SWIR 1 6900:1 SWIR 2: 2600:1 |  |
|  | VNIR: 525:1 SWIR 1 1100:1 SWIR 2: 1300:1 |  |
|  | Scanning Time |  |
|  | 100 milliseconds |  |
|  | 100 milliseconds |  |