Document Title: ASD FieldSpec4 Specifications

Rev Date: October 2025

| Layout Preview | English Text | Translated Text |
| --- | --- | --- |
|  | Specifications |  |
|  | The ASD FieldSpec4 spectroradiometer is the highest-performance spectral analysis system, designed for users who require the best possible data quality under real-world field conditions. Now produced and supported by SciAps Inc., FieldSpec4 continues the industry-standard legacy for remote sensing, environmental monitoring, and advanced geoscience research. |  |
|  | FieldSpec4 is purpose-built for applications that rely on natural illumination, including solar-based reflectance measurements and ground-truthing of airborne and satellite sensors. Its optical design enables reliable collection of low-signal data in challenging conditions where lighting, atmosphere, and surface properties cannot be controlled. |  |
|  | Highest-performing spectral analysis system |  |
|  | Best-in-class signal-tonoise performance |  |
|  | Factory-installed, fixed fiber-optic input |  |
|  | True post-dispersive spectroradiometer architecture |  |
|  | Spectral Range |  |
|  | 250 - 2500 nm |  |
|  | Key Specifications |  |
|  | Best-in-class signal-to-noise performance enabled by direct optical coupling to the diffraction gratings, delivering maximum sensitivity for low-light and solar-illumination measurements. |  |
|  | Factory-installed, fixed fiber-optic input to eliminate junction losses and ensure long-term optical stability for demanding field and research applications. |  |
|  | True post-dispersive spectroradiometer architecture with support for external light sources and factory radiometric calibration for absolute spectral radiometry and remote-sensing validation workflows. |  |
|  | As a true post-dispersive spectroradiometer, FieldSpec4 supports the use of external light sources—most importantly sunlight—making it ideal for building high-quality spectral libraries and validating hyperspectral imagery. This capability allows researchers to replicate the same illumination conditions experienced by overflight sensors. |  |
|  | For users working in variable environments, unknown materials, or complex atmospheric conditions, FieldSpec4 delivers the sensitivity, stability, and measurement flexibility required for accurate, defensible spectral results. |  |
|  | For more information, or to schedule a demonstration: |  |
| (BACK) |  |  |
|  | Specifications |  |
|  | Product |  |
|  | Spectral Range |  |
|  | Resolution |  |
|  | Noise Equivalent Radiance (NEdL) |  |
|  | VNIR SWIR 1 SWIR 2 |  |
|  | FieldSpec 4 Hi-Res NG |  |
|  | 250 - 2500 nm |  |
|  | 3 nm @ 700 nm 6 nm @ 1400/2100 nm |  |
|  | 1.0 × 10⁻⁹ W/cm²/nm/sr @ 700 nm 8.0 × 10⁻⁹ W/cm²/nm/sr @ 1400 nm 8.0 × 10⁻⁹ W/cm²/nm/sr @ 2100 nm |  |
|  | FieldSpec 4 Hi-Res |  |
|  | 250 - 2500 nm |  |
|  | 3 nm @ 700 nm 8 nm @ 1400/2100 nm |  |
|  | 1.0 × 10⁻⁹ W/cm²/nm/sr @ 700nm 1: 1.4 × 10⁻⁹ W/cm²/nm/sr @ 1400nm 2: 2.2 × 10⁻⁹ W/cm²/nm/sr @ 2100nm |  |
|  | FieldSpec 4 Standard-Res |  |
|  | 250 - 2500 nm |  |
|  | 3 nm @ 700 nm 10 nm @ 1400/2100 nm |  |
|  | 1.0 × 10⁻⁹ W/cm²/nm/sr @ 700 nm 1.2 × 10⁻⁹ W/cm²/nm/sr @ 1400 nm 1.9 × 10⁻⁹ W/cm²/nm/sr @ 2100 nm |  |
|  | ASD Fieldspec® 4 Instrument Details |  |
|  | FieldSpec 4 Hi-Res |  |
|  | Spectral sampling (bandwidth) |  |
|  | 1.4 nm @ 350–1000 nm, 1.1 nm @ 1001–2500 nm |  |
|  | Scanning time: |  |
|  | 100 milliseconds |  |
|  | Stray light: |  |
|  | VNIR 0.02%, SWIR 1 & 2 0.01% |  |
|  | Wavelength reproducibility: |  |
|  | 0.1 nm |  |
|  | Wavelength accuracy: |  |
|  | 0.5 nm |  |
|  | Maximum radiance: |  |
|  | VNIR 2X solar, SWIR 10X solar |  |
|  | Channels: |  |
|  | 2151 |  |
|  | Detectors: |  |
|  | VNIR detector (350–1000 nm) 512 element silicon array SWIR 1 detector (1001–1800 nm) graded index InGaAs photodiode, two stage TE cooled SWIR 2 detector (1801–2500 nm) graded index InGaAs photodiode, two stage TE cooled |  |
|  | Imput: |  |
|  | 1.5 m fiber optic (25° field of view). Optional narrower field of view fiber optics available. |  |
|  | Weight: |  |
|  | 5.44 kg (12 lbs) |  |
|  | Calibration: |  |
|  | Wavelength, reflectance, radiance*, irradiance*. All calibrations are NIST traceable (*radiometric calibrations are optional). |  |
|  | Computer: |  |
|  | Windows® 7 64-bit laptop (instrument controller) |  |
|  | Warranty: |  |
|  | One year full warranty including expert customer support |  |
|  | For service of ASD products, contact |  |
|  | ASD NOW PART OF THE SCIAPS FAMILY OF PORTABLE ANALYZERS. |  |