API Inspector Formula Workbook

Step-by-step worked examples with variable definitions for API 510, API 570, and API 653

Prepared by: Lockhart Training Services

Each worked example includes the equation, the meaning of every variable, the given data, a 
step-by-step solution, and multiple-choice answers.

| Section | Topics |
| --- | --- |
| API 653 - Tank Inspection | Minimum shell thickness, hydrostatic pressure, corrosion rate, remaining life, inspection interval |
| API 510 - Pressure Vessels | MAWP, required thickness, hoop stress, longitudinal stress, corrosion rate |
| API 570 - Piping | Corrosion rate, remaining life, required thickness, hoop stress, inspection interval |

API 653 - Tank Inspection Worked Examples

Problem 1. How to calculate the Minimum Shell Thickness for a storage tank.

Equation: t = 2.6(H - 1) DG / SE

Meaning of variables:

t = minimum required shell thickness, in.

H = liquid height above the point being evaluated, ft

D = tank diameter, ft.

G = specific gravity of the stored liquid.

S = allowable stress of the shell material, psi.

E = weld joint efficiency. 
Given:

D = 120 ft

H = 36 ft

G = 0.85

S = 21,000 psi

E = 1.0

Step-by-step solution:

- Substitute the values into the equation:

t = 2.6(36 - 1) x (120) x (0.85) / (21,000 x 1.0)

t = 9,282 / 21,000 = 0.442”

Final answer: t = 0.44 in (rounded to 2 decimal places)

Multiple-choice answers:

A. 0.44 in

B. 0.31 in

C. 0.52 in

D. 0.60 in

ANSWER: A

Problem 2. How to determine the Hydrostatic Pressure at the Tank Bottom

Equation: P = 0.433HG 
Meaning of variables:

P = hydrostatic pressure, psi.

H = liquid height, ft.

G = specific gravity of the liquid. 
Given:

H = 50 ft

G = 0.82

Step-by-step solution:

- Write the formula:

P = 0.433HG

P = 0.433HG.

P = 0.433 x 50 x 0.82

P= 17.753 psi

Multiple-choice answers:

A. 17.8 psi

B. 21.7 psi

C. 14.0 psi

D. 12.4 psi

ANSWER: A (rounded to one decimal point)

Problem 3. How to calculate the Corrosion Rate for a storage tank.

Equation: CR = (t1 - t2) / years

Meaning of variables:

CR = corrosion rate, in./yr.	nk

t1 = first thickness reading, in.

t2 = current thickness reading, in.

years = time between readings, yr.

Given:

t1 = 0.500 in in 2014

t2 = 0.420 in in 2024

yrs between readings = 2024 – 2014 = 10 years

Step-by-step solution:

CR= 0.500 - 0.420 = 0.080 in. / 10 yrs = 0.008 in/yr

Multiple-choice answers:

- 0.008 in/yr

- 0.010 in/yr

- 0.006 in/yr

- 0.004 in/yr

ANSWER: A

Problem 4. How to calculate the Remaining Life for a storage tank.

Equation: RL = (t_actual - t_required) / CR

Meaning of variables:

RL = remaining life, years.

tactual = current measured thickness, in.

trequired = minimum required thickness, in.

CR = corrosion rate, in./yr.

Given:

tactual = 0.420 in

trequired = 0.340 in

CR = 0.008 in/yr

Step-by-step solution:

RL = 0.420” - 0.340” / 0.008”

RL = 0.080” /0.008”

RL = 10 yrs

Multiple-choice answers:

- 10 years

- 8 years

- 12 years

- 6 years

ANSWER: A

Problem 5. How to calculate the Thickness Inspection Interval for the tank shell.

Equation: Interval = RL is the lesser of 15 yrs are RCA / 2N

Meaning of variables:

Thickness Interval = maximum inspection interval, years.

RL = remaining life, years.

RCA = Remaining corrosion allowance

N = Corrosion rate 
Given:

RL = 16 years

RCA = 0.125”

N = 0.004”

Step-by-step solution:

- Write the formula:

            - Thickness Interval = lesser of 15yrs or RCA / 2n

                - = 15 yrs or (0.125” / 2 x 0.004)

                - = 15 yrs or (0.125” / 0.008)

                - = 15 yrs or (15.65 years)

                - = 15 yrs

Final answer: Interval = 8 years

Multiple-choice answers:

A. 8 years (Correct)

B. 16 years

C. 10 years

D. 6 years

Answer: A

API 510 - Pressure Vessel Worked Examples

Problem 1. Maximum Allowable Working Pressure (MAWP)

Equation: MAWP = SEt / (R + 0.6t)

Meaning of variables:

MAWP = maximum allowable working pressure, psi.

S = allowable stress, psi.

E = weld joint efficiency.

t = wall thickness, in.

R = inside radius, in. 
Given:

- S = 20,000 psi

- E = 1.0

- t = 0.50 in

- R = 30 in

Step-by-step solution:

MAWP = SEt / (R + 0.6t)

MAWP = (20,000 x 1.0 x 0.50) / (30 + 0.6 x 0.50)

= 10,000 / 30.30

= 330 psi

Final answer: MAWP = 330 psi 
Multiple-choice answers:

A. 330 psi (Correct)

B. 410 psi

C. 275 psi

D. 520 psi

Problem 2. How to calculate the Required Vessel Shell Thickness

Equation: t = PR / (SE - 0.6P) 
Meaning of variables:

t = minimum required thickness, in.

P = internal pressure, psi.

R = inside radius, in.

S = allowable stress, psi.

E = weld joint efficiency.

Given:

- P = 300 psi

- R = 24 in

- S = 20,000 psi

- E = 1.0

Step-by-step solution:

t = PR / (SE - 0.6P)

t = (300 x 24) / (20,000 x 1.0 - 0.6 x 300)

t = 7,200 / 19,820 = 0.3633”

t = 0.36”

Final answer: t = 0.36 in

Multiple-choice answers:

A. 0.36 in

B. 0.25 in

C. 0.48 in

D. 0.60 in

Answer:   A

Problem 3. How to calculate the Hoop Stress for a vessel shell.

Equation: H = PR / t

Meaning of variables:

H = hoop stress, psi.

P = internal pressure, psi.

R = inside radius, in.

t = wall thickness, in.

Given:

P = 250 psi

R = 20 in

t = 0.40 in

Step-by-step solution:

H = PR/t

H = (250 x 20) / 0.40”

H = 5,000 / 040”

H = 12,500 psi

Final answer: Hoop stress = 12,500 psi

Multiple-choice answers:

A. 12,500 psi

B. 9,000 psi

C. 15,000 psi

D. 18,000 psi

ANSWER: A

Problem 4. How to calculate the Longitudinal Stress for a pressure vessel shell.

Equation: L = PR / (2t) 
Meaning of variables:

L = longitudinal stress, psi.

P = internal pressure, psi.

R = inside radius, in.

t = wall thickness, in.

Given:

P = 250 psi

R = 20 in

t = 0.40 in

Step-by-step solution:

L = PR / (2t)

L = (250 x 20) / (2 x 0.40)

L = 5,000 / 0.80

L = 6,250 psi

Final answer: Longitudinal stress = 6,250 psi

Multiple-choice answers:

A. 6,250 psi

B. 8,000 psi

C. 12,500 psi

D. 15,000 psi

ANSWER: A

Problem 5. What is the Pressure Vessel Corrosion Rate

Equation: CR = (t1 - t2) / years

Meaning of variables:

CR = corrosion rate, in./yr.

t1 = earlier thickness reading, in.

t2 = later thickness reading, in.

years = time between readings, yr. 
Given:

t1 = 0.625 in in 2018

t2 = 0.565 in in 2024 
Step-by-step solution:

CR = (t1 - t2) / years

CR = (0.625 – 0.565) / 6 yrs

CR = 0.060 / 6

CR = 0.010 in/yr

Final answer: CR = 0.010 in/yr

Multiple-choice answers:

A. 0.010 in/yr

B. 0.006 in/yr

C. 0.012 in/yr

D. 0.015 in/yr

ANSWER: A

API 570 - Piping Worked Examples

Problem 1. How to calculate the Corrosion Rate for piping.

Equation: CR = (t1 - t2) / years

Meaning of variables:

CR = corrosion rate, in./yr.

t1 = earlier thickness reading, in.

t2 = later thickness reading, in.

years = time between readings, yr.

Given:

t1 = 0.375 in in 2017

t2 = 0.335 in in 2025

years =  2025 – 2027 =  8 yrs

Step-by-step solution:

CR = (t1 - t2) / years

CR = (0.375” – 0.335”) / 8

CR = 0.04 / 8

CR = 0.005 in/yr

Final answer: CR = 0.005 in/yr

Multiple-choice answers:

A. 0.005 in/yr

B. 0.003 in/yr

C. 0.010 in/yr

D. 0.020 in/yr

ANSWER: A

Problem 2. How to calculate the Remaining Life for Piping

Equation: RL = (tactual - trequired) / CR

Meaning of variables:

RL = remaining life, years.

tactual = current measured thickness, in.

trequired = minimum required thickness, in.

CR = corrosion rate, in./yr.

Given:

tactual = 0.335 in

trequired = 0.260 in

CR = 0.005 in/yr

Step-by-step solution:

RL = (tactual – trequired) / CR

RL = (0.335 – 0.260) / 0.005

RL = 0.075 / 0.005

RL = 15

Final answer: RL = 15 years

Multiple-choice answers:

A. 15 years

B. 10 years

C. 20 years

D. 8 years

Answer: A

Problem 3. What is the Minimum Required Pipe Thickness

Equation: t = PD / [2(SE + PY)]

Meaning of variables:

t = minimum required thickness, in.

P = internal pressure, psi.

D = outside diameter, in.

S = allowable stress, psi.

E = weld joint quality factor or longitudinal joint factor.

Y = coefficient from the code table.

Given:

P = 300 psi

D = 12 in

S = 20,000 psi

E = 1.0

Y = 0.4

Step-by-step solution:

t = PD / [2(SE + PY)]

t = (300 x 12) / [2(20,000 x 1) + 300 x 0.4)]

t = 3,600 / 40,000 + 120

t = 3,600 / 40,240

t = 0.895” (rounded to 0.09”)

Final answer: t = 0.09 in

Multiple-choice answers:

A. 0.09 in

B. 0.14 in

C. 0.20 in

D. 0.32 in

ANSWER: A

Problem 4. Pipe Hoop Stress

Equation: H = PD / (2t)

Meaning of variables:

H = hoop stress, psi.

P = internal pressure, psi.

D = pipe diameter, in.

t = wall thickness, in.

Given:

P = 300 psi

D = 10 in

t = 0.25 in

Step-by-step solution:

H = PD / (2t)

H = (300 x 10) / (2 x 0.25)

H = 3,000 / 0.50

H = 6,000 psi

Final answer: Hoop stress = 6,000 psi

Multiple-choice answers:

A. 6,000 psi

B. 8,000 psi

C. 10,000 psi

D. 12,000 psi

ANSWER: A

Problem 5. How to determine the Thickness Inspection Interval for Piping

Equation: Thickness Interval = Lesser of Table 2 (for pipe class) or RL / 2

Meaning of variables:

Thickness Interval = maximum inspection interval, years.

RL = remaining life, years.

Class = piping class

Given:

RL = 18 years

Class = 2

Step-by-step solution:

Thickness Interval = Lesser of Table 2 (for pipe class) or RL / 2

Thickness Interval = lesser of 10 yrs per Table 2 of API 570 or 18 / 2

Thickness Interval = lesser of 10 per or 9 yrs

Thickness Interval = 9 yrs

Final answer: Interval = 9 years

Multiple-choice answers:

A. 9 years

B. 18 years

C. 12 years

D. 6 years

ANWER: