Tranquil Systems International	December 2025

528 Pioneer Parkway

Clare, Michigan 48617

Toll Free	800-631-0212

Website	www.tranquilsystems.com

E-mail	sales@tranquilsystems.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The CSI Construction Specifications Practice Guide.

This section must be carefully reviewed and edited by the Architect to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section numbers and titles are from MasterFormat 2012 Update.

10 22 19

DEMOUNTABLE PARTITIONS

Specifier Notes:  This section covers Tranquil Systems International “Tranquil Adapt Walls” with aluminum-trimmed, glass demountable partitions; floor-to-ceiling glass partitions with multiple configurations and privacy levels. Consult Tranquil Systems International for assistance in editing this section for the specific application.

GENERAL

SECTION INCLUDES

Floor-to-ceiling, “Adapt” aluminum-trimmed glass, demountable partitions.

RELATED REQUIREMENTS

Specifier Notes:  Edit the following list of related divisions as necessary.  Limit the list to divisions with specific information that the reader might expect to find in this section, but is specified elsewhere.

Division 8 – Openings:  Doors, door frames, and clerestory.

Division 26 – Electrical:  Raceway and boxes for electrical systems in internal panel cavities.

Division 27 – Communications:  Pathways for communications systems in internal panel cavities.

REFERENCE STANDARDS

American Society of Testing and Materials International (ASTM)

ASTM A 653 / A 653M – Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

ASTM C 1396 – Standard Specification for Gypsum Board

ASTM E 119 – Standard Test Method for Fire Tests of Building Construction and Materials.

ASTM E 84 – Standard Test Method for Surface Burning Characteristics of Building Materials.

ASTM E 90 – Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements.

ASTM E 413 – Classification for Rating Sound Insulation.

International Building Code (IBC)

IBC 2002 1607.14 – Structural Design, Live Loads, Interior walls and partitions

American Society of Civil Engineers (ASCE)

ASCE 7 – Minimum Design Loads for Buildings and Other Structures

PERFORMANCE REQUIREMENTS

Structural Performance: Provide demountable partitions capable of withstanding the effects of gravity loads and the following loads and stresses within limits and under conditions indicated:

Transverse-Load Capacity: Lateral deflection of not more than 1/120 (“Seclusion” or “Access” walls) and 1/175 (“Adapt” glass walls) of the overall span when tested using a uniformly distributed load of 5 psf (0.240 kN/m2) in accordance with IBC 2002 1607.14 or calculated by a registered engineer.

Seismic Performance: Provide demountable partitions capable of withstanding the effects of earthquake motions determined according to ASCE 7- Minimum Design Loads for Buildings and Other Structures.

Soil Type, Ground Motion Response Acceleration, and Structure Risk Category determine Seismic Design Category: Site specific.

“Tranquil Walls” internal X-bracing and wall kicker placement in accordance with calculations by a registered engineer.

Acoustic Performance: Provide demountable partitions with Sound Transmission Class (STC) ratings tested in accordance with ASTM E 90.

Specifier Notes:  Edit wall type and sound performance desired. Delete wall types not required.

“Adapt” aluminum-trimmed glass partitions: [Single pane offset, ¼” tempered, 26 STC] [Dual pane, ¼” tempered source side, ¼” tempered receive side, Glass-to-glass air space 1-3/8”, 30 STC] [Dual pane, ¼” tempered source side, 3/8” tempered receive side, Glass-to-glass air space 1-5/16”, 40 STC] [Single pane offset, 3/8” tempered, 27 STC] [Single pane offset, ½” tempered, 29 STC] [Dual pane, 3/8” tempered source side, ½” tempered receive side, Glass-to-glass air space 1-1/4”, 40 STC]

PREINSTALLATION MEETINGS

Specifier Notes:  Edit preinstallation meetings as necessary.  Delete if not required.

Convene preinstallation meeting [1 week]  [2 weeks] before start of installation of demountable partitions.

Require attendance of parties directly affecting work of this section, including Contractor, Architect, installer, and manufacturer’s representative.

Review materials, installation, cleaning, protection, and coordination with other work.

SUBMITTALS

Specifier Notes:  Edit submittal requirements as necessary.  Delete submittals not required.

Comply with Section 01 33 00 – Submittal Procedures.

Product Data:  Submit manufacturer’s product data, including installation instructions.

Shop Drawings:  Submit manufacturer’s shop drawings, including plans, elevations, sections, and details, indicating dimensions, tolerances, materials, components, fabrication, fasteners, hardware, finish, options, and accessories.

Indicate locations and sizes of doors and door frames.

Indicate locations and sizes of internal panel cavities for plumbing, electrical, and communications systems.

Samples:  Submit manufacturer’s sample of demountable partition panels.

Sample Size:  Minimum 6 inches by 6 inches.

Installer’s Project References:  Submit installer’s list of successfully completed demountable partition projects, including project name and location, name of architect, and type and quantity of partitions installed.

Care and Maintenance Instructions:  Submit manufacturer’s care and maintenance instructions, including:

Installing, removing, and relocating demountable partitions without damage.

Cleaning demountable partitions.

Warranty Documentation:  Submit manufacturer’s standard warranty.

Record Documentation:  Submit annotated shop drawings and specifications marked up to indicate actual products used and locations of work of this section.

SCHEDULING

Specifier Notes:  Edit order of floor coverings and suspended ceilings install. Directly affects how Tranquil Wall system is attached and supported.

Floor Coverings: [Install Tranquil wall system before installation of finished floor]  [Install Tranquil wall system after installation of finished floor]

Suspended Ceilings: [Install Tranquil wall system before installation of suspended ceiling]  [Install Tranquil wall system after installation of suspended ceiling]

QUALITY ASSURANCE

Installer's Qualifications:

Installer regularly engaged in installation of demountable partitions of similar type to that specified.

Employ persons trained for installation of demountable partitions.

Certified by manufacturer.

DELIVERY, STORAGE, AND HANDLING

Delivery Requirements:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

Storage and Handling Requirements:

Store and handle materials in accordance with manufacturer’s instructions.

Keep materials in manufacturer’s original, unopened containers and packaging until installation.

Store materials in clean, dry area indoors.

Do not store materials directly on floor.

Store materials on flat, level surface, raised above floor, with adequate support to prevent sagging.

Store materials out of direct sunlight.

Protect materials and finish during storage, handling, and installation to prevent damage.

PRODUCTS

MANUFACTURER

Manufacturer:  Tranquil Systems International, 528 Pioneer Parkway, Clare, Michigan 48617.  Toll Free 800-631-0212.  Website www.tranquilsystems.com. E-mail sales@tranquilsystems.com.

Specifier Notes:  Specify “Tranquil Adapt Walls” aluminum-trimmed, glass demountable partitions. Delete type of demountable partitions and/or panels not required.

GLASS DEMOUNTABLE PARTITIONS

“Tranquil Adapt Walls” Framing

Description:

Floor-to-ceiling demountable partitions.

Compact aluminum extrusion framing: 3-inch-wide by 1-1/4-inch tall.

Triple-track frame: Allows for single, dual, or triple panel configurations in track.

Glass panels framed with 1-1/4-inch stud or frameless with clear partition joint.

Framing at ceiling can be attached to ceiling grids, finished drywall headers, or standalone Tranquil post system.

Glass sliding doors: Pocket door slides within 3-inch-wide track. Accepts locking or non-locking ladder pulls or patch-mounted mortise sliding locksets for doors requiring positive latching for smoke control.

Aluminum or wood swing doors: Narrow, medium, and wide stile aluminum-framed glass doors with single or dual pane glass. Wood doors available in many grades and finishes. Industry standard cylindrical and mortise locksets, surface mount closers, electric strikes, and automatic door bottoms available.

Glass swing doors: Frameless glass with automatic closing patch fittings, locking and non-locking ladder pulls or patch-mounted cylindrical or mortise locksets. Electric strikes compatible with this style of door.

Aluminum door frame has door seal gasketing on sides and top.

Bulb seal glazing internally mounted in track below trim, no roll-in gasketing.

Finish trim installed after framing and panels are in place.

Aluminum Posts (3-inch square or 5-1/4-inch square): Horizontal and vertical posts used in wall design for freestanding or custom room configurations.

Cavities in Posts:  Facilitates installation of electrical wiring, receptacles and communications systems.

“Tranquil Adapt” Panels

Panel Dimensions:

Specifier Notes:  Specify panel height.  Panel heights are available from 48 inches to 120 inches in 1/8-inch increments.

Height: [_____ inches].

Specifier Notes:  Specify panel width.  Consult Tranquil Systems International for more information.

Width:  [_____ inches], plus or minus 0.005 inch.

Specifier Notes:  Specify glass or solid panels.  Delete panels not required.

Glass Panels:

Specifier Notes:  Specify single or dual pane, and glass thickness, type, and visibility.  Consult Tranquil Systems International for information regarding glass panel options, including specialized glass.

Pane Configuration: [Single centered] [Single offset] [Dual, 2-inch air space between panes].

Pane Thickness: [1/4-inch thick] [3/8-inch thick] [1/2-inch thick].

Glass Type: [Tempered].

Glass Visibility: [Clear] [Etched Frosted] [Etched Design] [Laminated Design] [Other: _____________].

Specifier Notes:  Vinyl privacy films are optional.  Consult Tranquil Systems International for information regarding opacity and designs.  Delete if not required.

Vinyl Privacy Films:  Applied after glass panel installation.

Opacity:  [____________].

Design:  [____________].

Solid Panels:

Specifier Notes:  Specify type of solid panels.  Consult Tranquil Systems International for information regarding solid panel options, including finishes and green panels.

Panel Configuration: [Single centered] [Single offset] [Dual] [Other: ________].

1/2-inch vinyl-wrapped gypsum. Vinyl Pattern and Color: [Specify].

1/2-inch Laminated panel, MDF core, prefinished. Finish: [Specify brand, color and sheen] [White markerboard double-sided] [White markerboard, magnetic, double-sided].

Other: [___________].

Trim:  Extruded aluminum, unique to pane configuration.

Specifier Notes:  Specify trim finish.  If powder coat is specified, choose color.

Finish:  [Clear anodized] [Powder coat color: ____________].

Doors and Hardware

Specifier Notes:  Specify type of doors. Delete doors and hardware not required. Consult Tranquil Systems International for information regarding door options, including finishes.

Sliding Glass Doors:

Glass: 3/8” Tempered.

Glass pocket door slides inside track, not surface-mounted, choose track: [Inside] [Outside] [Center, used only with dual pane sliding door where inside and outside tracks have fixed lites of glass].

Door Configuration: [Single] [Double].

Door Track Mechanism: [Soft Close Shock Absorption] [No Soft Close, Bumper Stop Only].

Handle Type: [Ladder pull] [Mortise lockset, for sliding doors needing positive latch, only works with single doors, door in inside track, requires post for latching].

If ladder pull, choose type: [Non-locking] [Locking].

If non-locking, choose length: [12”] [24”] [36”] [48”].

Non-locking pull color: [Brushed Stainless Steel] [Matte Black].

If locking, choose length and type: [48” floor locking SFIC] [48” floor locking URHD] [Custom length top locking URHD].

Locking pull color: [Brushed Stainless Steel] [Matte Black].

Mortise lockset brand: [Accurate].

Lock Functions: [Passage latch, non-keyed] [Office lock, keyed] [Storeroom, keyed, required with electric strike].

Lock Keying: [SFIC Cylinder, with core and key] [SFIC Cylinder, without core or key] [Other, specify cylinder, core, and key: ________].

Lockset Finish: [Satin Chrome, US26D] [Matte Black] [Other: ________].

Aluminum or Wood Swing Doors:

Type: [Solid core wood] [Hollow core wood] [Aluminum framed glass, single pane 1/4”] [Aluminum framed glass, dual pane 1/4”] [Other: ________].

Finish: [Wood: Enviroclad UV] [Aluminum: Clear Anodize] [Aluminum: Black Anodize].

If wood door, specify color: [_____________].

Handle Type: [Ladder pull] [Cylindrical lockset] [Mortise lockset].

If ladder pull, choose type: [Non-locking] [Locking].

If non-locking, choose length: [12”] [24”] [36”] [48”].

Non-locking pull color: [Brushed Stainless Steel] [Matte Black].

If locking, choose length and type: [48” floor locking SFIC] [48” floor locking URHD] [Custom length top locking URHD].

Locking pull color: [Brushed Stainless Steel] [Matte Black].

Cylindrical or mortise lockset brand: [Schlage] [Accurate] [Other: ________].

Lockset part number, including rose/escutcheon and lever style: [_____________]

Lock Functions: [Passage latch, non-keyed] [Entrance lock, keyed] [Office lock, keyed] [Storeroom, keyed, required with electric strike] [Other: ________].

Lock Keying: [SFIC Cylinder, with core and key] [SFIC Cylinder, without core or key] [Other, specify cylinder, core, and key: ________].

Lockset Finish: [Satin Chrome, US26D] [Matte Black] [Other: ________].

Hinges: [4.5” x 4.5” ball bearing mortise, US26D] [4.5” x 4.5” ball bearing mortise, L1 matte black] [Other: __________].

Surface Closer: [None] [Closer with 90 degree hold open] [Closer without hold open].

Surface Closer Finish: [Aluminum paint] [Black paint].

Automatic Door Bottom: [None] [ADB flush-mount inside door bottom rail].

Electric Strike: [None] [12/24V Fail safe/secure field selectable compact electric strike]. [Magnetic Lock, overhead].

Glass Swing Doors:

Type: [1/2” Tempered].

Patch type: [Non-closing, non-hold open patch fitting] [Hydraulic closing patch fitting]

Patch Color: [Brushed Stainless Steel] [Matte Black]

Handle Type: [Ladder pull] [Cylindrical lockset] [Mortise lockset].

If ladder pull, choose type: [Non-locking] [Locking].

If non-locking, choose length: [12”] [24”] [36”] [48”].

Non-locking pull color: [Brushed Stainless Steel] [Matte Black].

If locking, choose length and type: [48” floor locking SFIC] [48” floor locking URHD] [Custom length top locking URHD].

Ladder Pull Color: [Brushed Stainless Steel] [Matte Black].

Cylindrical or Mortise Lockset Brand: [Schlage] [Accurate] [CRL] [Other: ________].

Lockset Part Number, including rose/escutcheon and lever style: [_____________]

Lock Functions: [Passage latch, non-keyed] [Entrance lock, keyed] [Office lock, keyed] [Storeroom, keyed, required with electric strike] [Other: ________].

Lock Keying: [SFIC Cylinder, with core and key] [SFIC Cylinder, without core or key] [Other, specify cylinder, core, and key: ________].

Lockset Finish: [Satin Chrome, US26D] [Matte Black] [Other: ________].

Electric Strike: [None] [12/24V Fail safe/secure field selectable compact electric strike] [Magnetic Lock, overhead].

Door Stops:

Type, for door in open position: [None] [Floor mount] [Wall mount] [Frame mount].

Color: [Silver] [Matte Black].

Specifier Notes:  Delete “Adapt” Door Frames if Sliding Glass Doors is specified.

“Adapt” Door Frames:  Extruded aluminum, non-handed reversible door frames integrated into wall frame system. Includes door seal gasketing sides and top of frame.

Accessories

Electrical System:

Specifier Notes:  Specify modular or conventional electrical system. Delete if electrical not required.

Modular: Provide 3-inch or 5-1/4-inch vertical square posts for wiring and receptacles. 8 wires, 4 circuits, plug-and-play connections. Provided and installed by Tranquil Systems International and/or authorized installer.

Conventional:  Specified in Division 26.

EXECUTION

EXAMINATION

Examine areas to receive demountable partitions and verify field dimensions.

Verify surfaces to support demountable partitions are clean, dry, flat, level, plumb, square, stable, rigid, and capable of supporting the weight.

Notify Architect of conditions that would adversely affect installation or subsequent use.

Do not begin installation until unacceptable conditions are corrected.

INSTALLATION

Install demountable partitions in accordance with manufacturer’s instructions at locations indicated on the Drawings.

Install demountable partitions level, plumb, square, and straight.

Attach demountable partitions securely in place to supports.

Specifier Notes:  Edit the following sentence as necessary.  Delete if not required.

Install seismic bracing as indicated on the Drawings and as required to meet building code.

Repair minor damages to finish in accordance with manufacturer’s instructions and as approved by Architect.

Remove and replace with new material, damaged components that cannot be successfully repaired, as determined by Architect.

ADJUSTING

Adjust doors for proper operation in accordance with manufacturer’s instructions.

Adjust door hardware to operate smoothly without binding.

CLEANING

Clean demountable partitions promptly after installation in accordance with manufacturer’s instructions.

Do not use harsh cleaning materials or methods that could damage panels or finish.

Specifier Notes:  Delete the following sentence if this item is specified in Division 01 – General Requirements.

Remove debris associated with installation of demountable partitions and leave area in “broom clean” condition.

PROTECTION

Protect installed demountable partitions from damage during construction.

WARRANTY

10 Year Limited Warranty: We warrant each new Tranquil Systems International product of our manufacture to be free from defects in material and workmanship for ten (10) full years under normal use and service to the original buyers through one of our authorized Distribution partners. Restrictions apply. Please reference TSI Warranty for more detail.

END OF SECTION