Software Update for Smart Protect Plus via microSD card

(with the machine plugged in but powered off)

- Insert the microSD card into the microSD card slot at the	of the detector.

- Hold all three buttons until "UPD" is shown on the display.

- The software is automatically implemented on the device. Takes about 10 seconds.

- After successful updating, the detector will beep two times and will switch off automatically.

- Remove the microSD card and use the machine.

- Now calibrate the machine using the white card provided, instructions are overleaf.

*** Please note all Northern Ireland and Scottish banknotes need to be inserted face up and to the right of the input channel (as shown below) ***

CALIBRATION INSTRUCTIONS

Smart Protect Plus

[

EN

]

ratiotec GmbH & Co. KG | Tel.: +49 201 361 490 | Fax: +49 201 361 49 26 | Email: info@ratio-tec.de

2.

1.

3.

4.

5.

6.

Connect the Smart Protect Plus to the power

supply with the help of the power adapter.

Switch on the device by using the

ON / OFF

button and wait until

the device is ready.

Press the

ON / OFF button 4

times.

A20

is

shown on the display.

Carefully guide the white calibration paper

into the device. The device pulls the white

calibration paper and sends it back through

the front.

Repeat the process until the

calibration is complete and

the message

„Good“

is displayed. Switch

the device off by pressing the

ON / OFF

Button.

Turn on the device and check if the calibration

was successful. If not, please contact our

technical service.

Smart Protect Plus

To do a quick calibration of the Smart Protect

Plus you need following products:

Smart Protect Plus device

-

Power supply from Smart Protect Plus

-

An original white calibration paper

-

Please follow the steps below to make a short

calibration of the Smart

Protect Plus: