MV:									Permit Number:

Location - Name of Space / Tank:

Reason for entry:

- Permit Validity

The maximum period of validity is 12 hours subject to regular atmosphere checks.  For tankers that use N2 as inerting medium then the maximum permit validity is 8 hours.

Ensure that times entered for Personnel Statements and Atmosphere Checks are within the permit validity period.

This permit is valid from  Date:			Time:   From:             To:

- Prior Entry For multiple tanks (Chemical Tankers), SAF 11C replaces the pre entry atmosphere checks in this section.

Space 	When opened and unattended, physical restriction against entry in place

Space 	Ventilated by mechanical means

Space 	Has the space been cleaned where necessary

| - Pre Entry Atmosphere Checks Atmosphere tested (within 30 minutes of proposed entry time) |  |
| --- | --- |
| - O2 | - ≥20.8 %O2 Vol Value: |
| - Hydrocarbon | - ≤1% LEL Vol Value: |
| - Benzene** | - ≤0.5 PPM Value: |
| - H2S | - ≤2.5 PPM Value: |
| - CO | - ≤12.5 PPM Value: |
| - Toxic Gases | - ≤ 50% TLV of specific Gas Vol Value: |
| - ** Benzene content to be checked in compartments which have recently carried the product. |  |

Pipelines/Valves leading to enclosed space	Segregated/Blanked off/ Isolated

Electrical Power / Equipment	Isolated

Cargo/ Other Operations	Suspended

- Officer of Watch	Advised of planned entry

- Other Permits to Work	Issued

- Hot Work                           Aloft / Overside                    Energy Isolation

- Any other permits issued remain valid only if this permit (SAF 11)remains valid

Designated Link Man / Attendant at entrance	In constant attendance

Report Interval (every _____ minutes)	Agreed

SCAFFTAG Entry Kit / Lock-out Tag out	In use

Atmosphere checks (every 30 minutes)	Arranged

Lighting	Adequate

Equipment	In good working order

Ventilation	Continuous throughout period of permit

Routine entry to be completed during daylight hours	Yes

Working in space permitted as per VMS	Yes

- Safety

- Firefighting, first aid, rescue and resuscitation equipment at entrance	Available

- ELSA Sets	Available

- Gas detection Equipment inc. Personal Gas Monitors	Calibrated

- Gas detection Equipment inc. Personal Gas Monitors	Visually Inspected and Function Checked

- Communication System	Tested

- Emergency signals	Agreed

Emergency /evacuation procedures	Understood by all concerned

Appropriate PPE (inc. torches and Personal Gas Monitors)	Issued

Rescue harnesses (and lifelines, where practical)	Worn by those entering tank

Rescue Team identified	Yes

- Inspection Records

| - Equipment | - Gauge & Capacity of air supply |  |
| --- | --- | --- |
| - Breathing Apparatus Set 1 | - Bar/PSI |  |
| - Gauge & Capacity of air supply Checked - Low pressure audible Alarm Tested & operational - BA Face Mask Tested under positive pressure & not leaking |  |  |
| Breathing Apparatus Set 2 | - Bar/PSI |  |
| - Gauge & Capacity of air supply Checked - Low pressure audible Alarm Tested & operational - BA Face Mask Tested under positive pressure & not leaking |  |  |
| Breathing Apparatus Set 3 | - Bar/PSI |  |

- Gauge & Capacity of air supply Checked - Low pressure audible Alarm Tested & operational - BA Face Mask Tested under positive pressure & not leaking

| Breathing Apparatus Set 4 | - Bar/PSI |  |
| --- | --- | --- |

- Gauge & Capacity of air supply Checked - Low pressure audible Alarm Tested & operational - BA Face Mask Tested under positive pressure & not leaking

Note: When S.C.B.A is in use SAF 40 must also be used.

- Personnel Statement

The Permit Checklist (Sections A,B,C and D) has been completed and all found in order.

| Name: | Rank: Role: | Signature |
| --- | --- | --- |
|  | Responsible Officer |  |
| Date: | Time: |  |

To be signed by all personnel entering the space or monitoring operation.

| Name: | Rank: Role: | Signature |
| --- | --- | --- |
|  | e.g. Team Leader |  |
|  | e.g. OOW/EOW |  |
|  | e.g. Link Man/Attendant |  |
|  | e.g. Rescue team |  |

I have signed the above in agreement that:

- I have received instruction/permission from the Master or nominated responsible Officer to enter the enclosed space

- Checks have been made as per checklist above.

- I understand the duties and responsibilities of my role.

- I understand the communication procedures.

Radio   Visual  Verbal 	Audible  Rope Signal  Phone  Hand Signal  Other

- I understand the emergency and evacuation procedures.

- I am aware that the space must be vacated immediately in the event of ventilation failure or if atmosphere tests show a change from agreed safe criteria.

- I am familiar with the use of the equipment in use.

- Authorization

- Job Safety Analysis (Shipsure Reference:                       )  	Completed / Reviewed / Approved

- Permit Checklist 	Completed

- Toolbox talk to be held on site prior to entry	Confirmed

- Initial Atmosphere Checks ........................... Completed and safe to enter……………………………. Confirmed

Entry to Space	Authorized

- I authorize the entry to the enclosed space listed above:

- Name:           ______________________ (Master)

- Signature:     ______________________

- Date:             ______________________

- Time:             ______________________

- Ensure permit displayed at enclosed space entrance

- Permit Closure

- Work in Enclosed Space 	Completed

- Personnel	Clear from Enclosed Space

- Tools used	Removed from enclosed Space

Entry to Space	Secured

OOW	Informed of completion of Job

Permit to enter enclosed space	Closed

- Responsible Officer Name              ______________________

- Signature                                            ______________________

- Date                                                   _______________________

- Time                                                  ______________________

Completed

- Additional Comments: - Method of Rescue - Tripod Winch Block & tackle Other

H.	Atmosphere Checks

| Atmosphere Requirements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Oxygen ≥20.8% | LEL < 1% | Benzene** 0.5 ppm | H2S 2.5 ppm | CO 12.5 ppm | Toxic Gas <50% TLV |
| ENTRY LOG - Activity column to record all persons in or out of space, communications and atmosphere checks. Total In should show total number in tank at any time. Full atmosphere checks are to be made for re-entry after any break in work. |  |  |  |  |  |

| Time | Activity (Detail on table the names of persons entering tanks alongside the time.) | %O2 | %LEL | Toxic Gas (enter name in column) |  |  |  | *Entering or leaving the tank (strike through where N/A) | Total Persons In | Initial |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Benzene | H2S | CO |  |  |  |  |
|  | Initial Pre-entry atmosphere checks |  |  |  |  |  |  |  |  |  |
|  | Initial entry local atmosphere check |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  | Test frequency not to exceed 30 minutes |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |
|  |  |  |  |  |  |  |  | Entering / Leaving* |  |  |