Acoustical Banner Specification for 1 1/2” Guilford FR 701 Banner

SECTION 09800 – ACOUSTICAL CATENARY BANNER

PART 1 – GENERAL


1.1
SUMMARY

A. This section includes the following:

1. Acoustical Catenary banners.

1.2 SUBMITTALS

A. Shop Drawings: Show fabrication and installation details for ceiling baffles, including plans, elevations, sections, details and attachment to other work.

B. Submittals:  Furnish 8 1/2” x 11” samples, showing manufacture full range of colors, texture and patterns available for each type of acoustical baffle specified.

C. Product Test Report: From a qualified testing agency indicating ceiling baffles comply with requirements.

D. Qualification Data: For firms specified in “Quality Assurance” Article to demonstrate their capabilities and experience.

E. Product Certificates: Signed by manufacturer certifying that the products furnished comply with requirements.

1.3 QUALITY ASSURANCE

A. Manufacturer Qualification: Manufacture shall have a minimum of 5 years experience in production of specified products and shall furnish supporting documentation showing completed jobs of approximately the same size and scope.

B. Fire Test Reports: Provide ceiling banner with the following surface- burning characteristics as per CPAI 84.

C. Acoustical Test Report: Provide acoustical test report from a qualified testing agency indicating ceiling baffles meets 14.3 sabins/banner per C423-90a.

1.4 DELIVERY, STORAGE, AND HANDLING

A. Protect Ceiling baffles from excessive moisture when shipping, storing, and handling.  Deliver in unopened skids and store in a dry place with adequate air circulation.  Do not delivery material until all wet-work has been completed.

PART 2 – PRODUCTS

1.1 CEILING BANNER, GENERAL

A. Ceiling banner shall be 1 1/2” thick – 1.6 lb. per cubic foot density fiberglass.   Manufactures standard banner construction, sewn edges, banner manufactures standard full line of fabric.

1. Products

a. Model Catenary Banner Guilford FR 701

Sound Seal 

Agwam, MA.  USA

Ph. (800) 569-1294

Fax (413) 789-4444

2. Facing Material:  Guilford FR 701 fabric

3. Baffle Thickness: nominal 1 1/2” thick

4. Noise Reduction: 14.3 sabins/banner

5. Baffle Width: 48 inches

6. Baffle Length: As Specified

1.2 CONSTRUCTION

A. The acoustical ceiling banner product shall be supplied in 48” wide banner with lengths as indicated.  All edges will be sewn with grommets for mounting.

1.3 MOUNTING

A.
Manufactures standard accessories for securely mounting baffles, of type and size indicated and complying with the following requirements:


1. Grommets shall be installed to facilitate mounting on either the short of long edge of the baffle, as indicated on the drawing.

2.4
ACOUSTICAL PERFORMANCE

Sound Absorption: Per ASTM C423-90a

Sabins/Baffle  per Octave Band Center Frequency (Hz)

125
250
500
1000
2000
4000





2.85
7.25
13.7
18.10
18.0
16.9

2.5
FLAMMIBILITY RATING

A. All components shall have a Class A Flammability rating per  ASTM E-84.

PART 3 – EXECUTION

3.1 INSTALLATION

A. Install Ceiling banner in locations indicated, top edges level and in alignment with other baffles.   Comply with manufactures written instructions for installation of baffles using type of mounting accessories indicated or, if not indicated, as recommended by manufacture.

3.2 CLEANING

A. After completion of installation of banner, remove dust and other foreign material according to manufactures written instructions.

B. Remove surplus material, rubbish, and debris resulting from banner installation, on completion of the work, and leave areas of installation in a neat and clean condition.

