Interior ADA compliant room signage

Submittal requirements

- Manufacturers must submit 3 references showing products for projects completed within the previous 5 years.

- Submit manufacturer’s technical data and installation method for each type of sign required.

- Submit drawings showing all construction and graphic details including sign sizes, letterforms, letter height, and material.

- Submit one full size sample sign of each sign type, including method of attachment. If approved, the sample will be utilized as a completed component of the project.

Storage and transport requirements

- Package signs adequately to prevent damage during transport, handling, storage and installation.

- Handle signs in accordance with manufacturer’s instruction.

Compliance requirements

- All signs are required to comply with ADAAG and ANSI/ICC A117.1 and any additional local applicable building codes and regulations unless otherwise indicated. In the event of specification conflicts, signage will comply with the most comprehensive and specific requirements.

- Signage will comply with the 2010 Standards for Accessible Design (ADAAG). Requirements include, but are not limited to:

  - Tactile copy will be uppercase, 1/32” raised characters minimum, in a sans serif font that is not italic, script, oblique or highly decorative.

  - The stroke width of uppercase “I” will be 15% maximum of the character height. The width of the uppercase “O” will be 55% minimum to 110% maximum of the height of uppercase “I.”

  - Tactile copy height will be 5/8” minimum to 2” maximum. If additional visual characters are provided, raised characters will be ½” minimum and contrast rules will not apply.

  - The difference between tactile letters will be 1/8” minimum to 4 times maximum the width of “I.”

  - Spacing between baselines of tactile text will be 135% minimum to 170% maximum the height of “I.”

  - Tactile text will have a corresponding Grade II Braille translation directly below the body of text and 3/8” away from other raised and tactile elements.

  - Sign and sign graphics will have a non-glare finish and high-contrast appearance. A 70% minimum contrast between the sign graphics and background color is recommended for high contrast compliance.

  - Sign pictograms must sit within a 6” vertical field and their corresponding text descriptions will be included directly below the pictogram.

Products

Approved manufacturers

- Provide products from Simply Brandable Signs, available at www.simplybrandablesigns.com, (901) 861-3001, or approved “equal manufacturer” only. Other manufacturers must submit any proposed signage products to Architect at least 10 days prior to the bid date for approval to qualify for consideration as an “equal manufacturer.”

- Products under this section to be produced by an experienced manufacturer regularly engaged in work of this type and scope for a minimum of 5 years.

Product details

- All signs to be manufactured using the Simply Brandable Signs’ “The Underscore Collection” sign family specifications.

- Signs shall be 2-ply consisting of a faceplate and an extended backer that protrudes ½” out from the bottom edge of the faceplate.

- Faceplate Material shall be first surface painted Novacryl PT series 1/8” clear photopolymer with Polyethylene Terephthalate Glycol (PETG) base.

- Backer Material shall be 1/8” first surfaced painted clear extruded Polymethyl Methacrylate (PMMA) glare acrylic.

- Faceplate Color to be chosen from manufacturer’s selection.

- Typography shall be within Gotham family and following ADA-compliance standards.

- Text Color to be chosen from manufacturer’s selection with ADA-compliance standards in mind.

- Extended Backer Color to be chosen from manufacturer’s selection.

- Panel Edge Accent Color shall match the chosen Backer Color.

- Panel Sizes include but are not limited to:

    - Restroom/Stair overall sign size shall be 6” x 8 ½”

      - Restroom/Stair sign Backer shall be 6”  x 8 ½”

      - Restroom/Stair sign Faceplate shall be 6” x 8”

    - Small Core Room Identification overall sign size shall be 8 x 4 ½”

      - Small Core Room ID sign Backer shall be 8” x 4 ½”

      - Small Core Room ID sign Faceplate shall be 8” x 4”

    - Large Core Room Identification overall sign size shall be 8” x 6 ½”

      - Large Core Room ID sign Backer shall be 8” x 6 ½”

      - Large Core Room ID sign Faceplate shall be 8” x 6”

    - Window signs with insert housing pocket overall sign size shall be 6” x 6 6 ½”

      - Window sign Backer shall be 6” x 6 ½”

      - Window sign Faceplate shall be 6” x 6”

- Corner Radius of all signs shall be 0” or “square.”

Installation requirements

- Do not begin installation until all signage has been manufactured and prepared for installation.

- If signage preparation is completed by a third-party installer, notify architect of any instances of unsatisfactory preparation before installation.

- Clean all sign surfaces before installation.

- Prepare surfaces according to manufacturer’s recommendation to achieve best results for signage installation

- Install in accordance with manufacturer’s instructions.

- Products to be squared and leveled on walls and shall comply with the 2010 Standards for Accessible Design (ADAAG) and any applicable local regulations or codes. Requirements include, but are not limited to:

  - Panel signage with tactile text and braille shall be installed at a height where the baseline of the lowest line of tactile characters measures at 48” minimum above the ground and the baseline of the topmost line of tactile characters measures 60” maximum above the ground.

  - Panel signage with tactile text and braille shall be installed alongside the relevant door on the latch side. In the instance of double doors wherein both doors are active, signage shall be installed to the right of the righthand door. When no space is available at the latch side of a single door or on the right of a pair of double doors, sign shall be installed on the nearest adjacent wall.

  - Panel signage with tactile text and braille shall be installed within a clear floor space of 18” x 18” minimum, centered on the tactile characters, and beyond the arc of the door swing between the closed and 45 degree opening position of the door.

- Installation method shall include either or both of the following according to environmental factors:

  - Concealed double-sided vinyl tape.

  - Silicon adhesive.

- Provide adhesive backed vinyl decals matching sign panel color to be installed behind signs going on glass.

Protection

- Protect products from damage and debris until substantial completion of project.

- Touch-up, repair, or replace damaged products before substantial completion of project.