| SUBJECT: Parapet Wall inspection | REFERENCE: |
| --- | --- |
| DEPARTMENT: Engineering PM |  |
|  | EFFECTIVE: June11 2024 |
| APPROVED BY: | REVISED: |

To be in accordance with the requirements of section 28-301.1.1 of the Administrative Code for the City of New York, the building owner will have an observation performed in accordance with this subdivision. Owners must maintain the observation reports for at least six (6) years and must make such reports available to the Department upon request.

1.  The observation must be performed annually by a person competent in inspecting parapets. For the purposes of this subdivision. this includes. but is not limited to. a bricklayer. building superintendent, handyman, mason or a person in a similar construction-related trade, architect, engineer. an inspector working for a New York State-authorized insurance company, a New York State-authorized building inspector or any other individual capable of identifying hazards on the parapet.

2.  The observation must include close-up inspections of the entire parapet. The close-up inspection may be conducted from a fire escape or roof.

3.   The observation must include. but not be limited to:

- A determination that the parapet is plumb by a horizontal distance within one-eighth of its cross-sectional thickness in any location.

- A determination of whether there is excessive deterioration, including but not limited to. displacement, horizontal or diagonal cracks, missing or loose bricks or coping stones, deteriorated mortar joints, spalling, or rot.

- A determination that appurtenances such as telecommunications equipment. railings. roof access rails, gooseneck ladders and handrail attachments for fire escapes. and signs. have been installed and maintained in a stable condition.

# I.	Notification

If the parapet exhibits a hazardous or unsafe condition:

i	The person performing the observation must immediately notify the Department of Buildings: and

ii	The owner must immediately install public protection. such as erecting sidewalk sheds, fences, and safety netting, as may be required to secure the safety of the public. and have the unsafe condition remedied. All unsafe conditions must be corrected within ninety (90) days from the notification to the Department. The public protection must remain in place until the unsafe condition is remedied.

Date of inspection:________________________________

| # Building Address | # Inspector Contact Information |
| --- | --- |
| # Phone |  |
| # Title | # Affiliation/Business name |

# Insert roof/parapet plan here:

| # Material | # Height | # Thickness | # Other |
| --- | --- | --- | --- |

# General conditions noted:
______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

# Unsafe conditions found:

# ________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

# Corrections Completed:
______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

# Attach dated Photos of the areas inspected and conditions found.