🌱

JERSEY SMALLHOLDER

Carbon & Net Zero Plan

Measure, reduce and track the carbon footprint of your holding

Supports LEAF Marque & Red Tractor assurance  •  Aligned to Jersey’s Carbon Neutral Roadmap

[Holding / Small Farm Name] [Address, Parish, Jersey] Size: [__ vergees] ¦ Baseline year: [YYYY] ¦ Plan covers: [YYYY–YYYY] Completed by: [Name] ¦ Date: [DD/MM/YYYY] ¦ Next review: [DD/MM/YYYY]

🇯🇪 The single most important fact for Jersey smallholders: Jersey’s electricity (from JEC) is already nearly carbon-free — about 25g CO₂e per kWh The UK mainland grid emits about 233g CO₂e per kWh — nearly ten times more This means: on a Jersey holding, diesel, heating oil, LPG and livestock are your biggest carbon sources — NOT your electricity Switching from diesel or oil TO electric equipment is one of the most effective things you can do on a Jersey holding Jersey’s Carbon Neutral Roadmap commits to -68% emissions by 2030 and net zero by 2050 (Paris Agreement, extended to Jersey May 2022)

Keep this plan with your farm records and review it every year.

Bring it to any LEAF Marque or Red Tractor inspection.

# How to Use This Plan

This plan helps you understand, measure and reduce the greenhouse gas (GHG) emissions from your Jersey smallholding. It is written in plain English — you do not need to be a scientist or consultant to complete it. Work through each section, filling in the figures that apply to your holding. Write N/A for sections that don’t apply.

The key steps are:

- Understand where your emissions come from (Section 2)

- Collect the data you need, even if estimates at first (Section 3)

- Calculate your carbon footprint in the summary table (Section 4)

- Identify where your carbon is being stored on the holding (Section 5)

- Set your targets and actions (Section 6)

- Review once a year and update your figures (Section 7)

✅ What is a carbon footprint? • A carbon footprint is the total amount of greenhouse gases produced by your activities, expressed in kg or tonnes of CO₂ equivalent (CO₂e) • Greenhouse gases include carbon dioxide (CO₂), methane (CH₄ — from livestock and manure) and nitrous oxide (N₂O — from fertilisers and soils) • CO₂ equivalent (CO₂e) converts all gases to a single number: methane is 28× more potent than CO₂ per kg; nitrous oxide is 273× more potent • Net zero means balancing what you emit with what you store (sequester) in soil, trees and hedgerows • Even a small holding can make a real difference — and many of the best actions also reduce your costs

Don’t worry if your first footprint is rough — an honest estimate is far more useful than no measurement at all. Accuracy improves year on year as you gather better data.

# Section 1 – About Your Holding

| Holding name | What do you call your place? |
| --- | --- |
| Address & Parish | Full address including parish and postcode |
| Total size (vergees / acres) | 1 vergee = 0.44 acres / 0.177 hectares |
| Main activities | e.g. market garden, polytunnels, chickens, goats, pigs, dairy goats, bees, orchard, vines, mixed |
| Livestock (type and numbers) | e.g. 4 dairy goats, 20 laying hens, 2 pigs — needed for methane estimate |
| Do you use irrigation? | Y/N — if yes, how powered? (diesel pump / electric pump / gravity) |
| Do you use a cold store? | Y/N — if yes, what refrigerant? (check the nameplate on the unit) |
| JEC electricity customer? | Y/N — almost certainly yes; grid carbon: ~25g CO₂e/kWh |
| Any solar panels? | Y/N — if yes, capacity (kWp) and annual output (kWh from inverter display) |
| Plan baseline year | The year you are measuring as your starting point — e.g. 2024 |
| Plan period | e.g. 2024–2030 — aligned to Jersey’s 2030 target |
| eco-active member? | Y/N — gov.je/ecoactive for free carbon tools and support from Government of Jersey |
| LEAF Marque / Red Tractor? | Y/N — if yes, annual carbon evidence required at audit |
| Completed by / date | Your name and the date you filled this in |

# Section 2 – Where Do Your Emissions Come From?

Before you measure anything, it helps to understand the main sources of greenhouse gases on a small Jersey holding and how they compare. The traffic-light layout below reflects the Jersey context — it is deliberately different from what you might read in a UK mainland guide.

| 🚦 | Source | Why it matters on your Jersey holding | How to reduce it |
| --- | --- | --- | --- |
| 🔴 | Diesel, petrol, heating oil, LPG | Usually the BIGGEST source for a small Jersey holding; every litre of diesel = ~2.5 kg CO₂e | Use less; switch to electric; insulate buildings |
| 🔴 | Livestock methane (if you keep ruminants) | Dairy goats, sheep or cattle produce methane through digestion; significant even in small numbers | Improve feed quality; fewer animals if feasible |
| 🔴 | Fertiliser – nitrous oxide from the soil | N₂O is 273× more potent than CO₂; released when nitrogen fertiliser is applied | Test soil; apply only what crops need; use slow-release forms |
| 🟡 | Bought-in fertiliser manufacture | Nitrogen fertiliser takes a lot of energy to make; ~4.3 kg CO₂e per kg N bought | Precision application; use compost/manure to replace some synthetic N |
| 🟡 | Bought-in animal feed and bedding | Embedded emissions from growing and transporting feed; moderate source | Grow/source locally; reduce waste |
| 🟢 | JEC electricity | VERY LOW in Jersey — ~25g CO₂e/kWh vs 233g on GB grid; switching to electric uses this advantage | Switch oil/diesel equipment to electric to use this clean power |
| 🟢 | Packaging / plastics | Moderate; plastic mulch and polytunnel film have embedded carbon | Use biodegradable or reusable; buy in bulk to reduce transport |

🇯🇪 The Jersey electricity advantage — understand this before anything else JEC imports 95% of Jersey’s electricity from nuclear and hydro sources in France via undersea cables This makes JEC electricity almost ten times cleaner than the UK grid per unit of energy Practical examples for your holding: • Running an electric irrigation pump instead of a diesel pump: saves ~2.5 kg CO₂e per litre of diesel replaced • Switching a heating oil boiler to an air-source heat pump: large saving per year (Gov of Jersey offers up to £5,000 via Low Carbon Heating Incentive) • Electric vehicle instead of petrol: saves ~2.3 kg CO₂e per litre of petrol replaced • Installing solar PV: reduces already-low Scope 2 further; but the biggest win is using clean JEC electricity instead of diesel/oil

# Section 3 – Data Collection: What to Record

The table below walks you through every emission source that could apply to a small Jersey holding. For each one, fill in how much you use in a year and use the emission factor to work out roughly how many kg CO₂e that produces. Leave a row blank if it doesn’t apply to you.

💡 Tip: start with your invoices Your fuel supplier (Jersey Fuel, Grays, Claude Sykes, Island Fuels) will have delivery records showing litres purchased Your JEC electricity bills show kWh used each quarter — add them up for a full year Your fertiliser invoices show kg of product bought — check the N content on the bag (%N) Your farm diary or field records should show livestock numbers through the year Even estimates are fine for the first year — you can refine them as you go

| What Are You Using? | What to Record & Where to Find It | Jersey Emission Factor | Your Amount (baseline year) | kgCO₂e (calc or estimate) | Main Action to Reduce This |
| --- | --- | --- | --- | --- | --- |
| Red diesel / gas oil (tractors, pumps, generators) | Delivery invoices from fuel supplier; litres purchased per year | 2.56 kgCO₂e per litre |  |  | Switch pumps/machinery to electric |
| Heating oil (kerosene) – polytunnel, building heating | Delivery invoices; litres per year | 2.54 kgCO₂e per litre |  |  | Air-source heat pump (Gov incentive up to £5k) |
| LPG – glasshouse, weed burner, heating | Delivery invoices or bottle receipts; litres or kg per year | 1.61 kgCO₂e per litre (2.94/kg) |  |  | Switch to electric alternatives |
| Petrol – quad bike, small engines, chainsaw | Fuel receipts; litres per year | 2.31 kgCO₂e per litre |  |  | Electric alternatives where feasible |
| White diesel (DERV) – van / car used for the holding | Fuel receipts; litres per year | 2.56 kgCO₂e per litre |  |  | Electric vehicle; JEC Evolve chargers |
| JEC electricity – all farm use | JEC quarterly bills; kWh per year (sum 4 bills) | 0.025 kgCO₂e per kWh (very low!) |  |  | Already clean; switch oil/diesel TO electric |
| Solar PV generated on site (deduct from above) | Inverter display or monitoring app; kWh per year | −0.025 kgCO₂e per kWh (credit) |  |  | Good — maintain panels, consider expanding |
| Dairy goats or sheep – enteric methane | Livestock numbers × days on holding per year | Goats: ~0.9 kgCO₂e/head/day; Sheep: ~0.27/head/day |  |  | Improve forage quality; avoid overstock |
| Chickens or pigs – manure N₂O | Livestock numbers; manure managed per year (approx tonnes) | Poultry: ~1.6 kgCO₂e/head/year; Pigs: ~0.3/head/year |  |  | Compost manure; avoid bare soil spreading |
| Cattle (if any) – enteric methane | Head × days on holding per year | Beef: ~3.3 kgCO₂e/head/day |  |  | Improve grazing management; forage quality |
| Synthetic N fertiliser applied | Fertiliser invoices; kg of nitrogen (N) applied (check %N on bag) | ~6.5 kgCO₂e per kg N applied (manufacture + soil N₂O) |  |  | Soil test; slow-release N; reduce rate |
| Organic N from bought-in manure / compost | Quantity bought in (tonnes); estimate N content | ~1.5 kgCO₂e per kg N (lower than synthetic) |  |  | Source locally; compost on-site instead |
| Cold store refrigerant top-up (if any) | Engineer log; kg of refrigerant added and gas type | R404A: 3,922 kgCO₂e/kg; R134a: 1,430; R290: 3/kg |  |  | Log all top-ups; replace with low-GWP gas at next service |
| Plastic mulch / polytunnel film bought | Purchase records; kg per year | ~3.0 kgCO₂e per kg |  |  | Biodegradable mulch; extend film life |
| Bought-in animal feed (concentrate) | Purchase records; tonnes per year | ~0.7 tCO₂e per tonne (estimate) |  |  | Grow/source locally; reduce waste |
| Packaging materials (bags, boxes, trays) | Purchase records; kg per year | ~0.8–3.0 kgCO₂e per kg (varies) |  |  | Bulk buying; reusable / returnable packaging |

Emission factors above are indicative, based on DEFRA/BEIS Greenhouse Gas Conversion Factors and the JEC annual carbon intensity report. The Cool Farm Tool (coolfarming.org) will calculate most of these automatically and is free to use.

# Section 4 – Your Annual Carbon Footprint Summary

Once you have filled in the data table in Section 3, transfer the totals here. This gives you your farm’s annual carbon footprint in a single table. Track it year on year to see the trend.

📈 How to calculate kgCO₂e Multiply your annual amount by the emission factor from Section 3 Example: 500 litres of red diesel × 2.56 = 1,280 kgCO₂e Example: 50 kWh of JEC electricity × 0.025 = 1.25 kgCO₂e (tiny!) Example: 5 dairy goats for 365 days: 5 × 365 × 0.9 = 1,643 kgCO₂e If you prefer to use a free tool, the Cool Farm Tool at coolfarming.org does all the maths for you 1,000 kgCO₂e = 1 tonne CO₂e (tCO₂e)

| Emission Source | Baseline Year (kgCO₂e) | Year 2 | Year 3 | Year 4 | Trend ↑↓ | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Diesel / petrol (tractors, vehicles, machinery) |  |  |  |  |  |  |
| Heating oil / LPG (buildings, glasshouse) |  |  |  |  |  |  |
| Livestock – enteric methane (if applicable) |  |  |  |  |  |  |
| Livestock – manure management (if applicable) |  |  |  |  |  |  |
| Fertiliser N applied – soil N₂O |  |  |  |  |  |  |
| Refrigerants (cold store top-ups, if any) |  |  |  |  |  |  |
| TOTAL SCOPE 1 (direct emissions) |  |  |  |  |  |  |
| JEC electricity purchased |  |  |  |  |  |  |
| TOTAL SCOPE 2 |  |  |  |  |  |  |
| Bought-in fertiliser manufacture |  |  |  |  |  |  |
| Bought-in feed / bedding (if applicable) |  |  |  |  |  |  |
| Bought-in packaging / plastic mulch |  |  |  |  |  |  |
| TOTAL SCOPE 3 (estimate) |  |  |  |  |  |  |
| GROSS TOTAL EMISSIONS |  |  |  |  |  |  |
| LESS: estimated sequestration (soil, trees, hedges) |  |  |  |  |  |  |
| NET CARBON POSITION |  |  |  |  |  |  |

Colour guide: orange rows = Scope 1 (direct emissions from your holding); green rows = Scope 2 (JEC electricity — very low in Jersey); blue rows = Scope 3 (embedded in things you buy). The NET CARBON POSITION is your gross total minus what you sequester in soil, trees and hedgerows.

## How does your footprint compare?

| Your holding gross footprint (kgCO₂e per year) | [Enter from table above] |
| --- | --- |
| Footprint per vergee (kgCO₂e / vergee) | [Gross total ÷ vergees farmed] |
| Footprint per unit of production | [e.g. kgCO₂e per dozen eggs, per kg salad, per litre milk] |
| On-farm sequestration (kgCO₂e credit, Section 5) | [Enter from Section 5] |
| Net carbon position | [Gross total minus sequestration — target: as low / negative as possible] |

# Section 5 – Carbon You Are Already Storing (Sequestration)

Every smallholding has the potential to store carbon in its soil, hedgerows, trees and permanent grass. This stored carbon — called sequestration — counts against your emissions, reducing your net carbon position. Understanding what you have is the first step to increasing it.

🇯🇪 Jersey-specific: earthen banks and valées are a hidden asset Jersey’s earthen banks (talus) are dense with root biomass and soil organic carbon — each well-managed bank stores significant carbon Banks are protected under Jersey’s Island Plan 2022–25, Policy NE3 — you cannot remove them anyway; but managing them well increases their carbon value Valée woodland along stream valleys is particularly carbon-dense; even a small area stores a disproportionate amount Traditional Jersey cider apple orchards with veteran trees hold large amounts of above-ground carbon in their trunks and branches Permanent grassland sequesters more soil carbon than regularly cultivated ground — even a small permanent paddock counts Contact the National Trust for Jersey (01534 483193) or Birds on the Edge (birdsontheedge.org) for free advice on maximising your holding’s natural capital

## 5.1 What to Record

| Earthen banks – total length (metres) | [Measure or estimate; well-vegetated banks store ~0.5–2 kgCO₂e per linear metre per year] |
| --- | --- |
| Earthen banks – condition | [Good (dense veg) / Poor (sparse/eroded) / Mixed — target: Good] |
| Trees and hedgerow trees – number (approx) | [Count significant trees; each mature tree ~2–10 kgCO₂e/year sequestered] |
| Woodland / copse / orchard area (vergees) | [Enter; young woodland ~0.4 tCO₂e/ha/yr; mature woodland ~2–4 tCO₂e/ha/yr] |
| Permanent grassland (vergees) | [Enter; permanent grass sequesters more than cultivated soil] |
| Soil organic matter (SOM) % – last measurement | [From soil test result; increasing SOM = sequestration happening] |
| Date of last soil test | [DD/MM/YYYY; aim to test every 3–5 years] |
| Ponds / wetland (if any) | [Area in vergees; managed wetland sequesters carbon and boosts biodiversity] |
| Estimated total sequestration (kgCO₂e/year) | [Use Farm Carbon Cutting Toolkit (farmcarbontoolkit.org.uk) for guidance; enter here] |

## 5.2 Actions to Increase Sequestration

| Action | What it Does | Who Can Help in Jersey | Target Date |
| --- | --- | --- | --- |
| Add cover crops between growing seasons | Builds soil organic matter and prevents N₂O loss from bare soil | Farm Jersey / RJA&HS agronomist |  |
| Apply compost or home-made FYM | ~0.3 tC per 10 tonnes applied; also reduces bought-in fertiliser | Make your own; or from Parish compost schemes |  |
| Plant new trees on boundaries or valées | ~0.4–2 tCO₂e/ha/yr as trees mature; also biodiversity | National Trust for Jersey; Birds on the Edge; free trees sometimes available |  |
| Manage earthen banks well — cut no more than once every 2–3 years | Denser vegetation and root biomass = more carbon stored | Island Plan NE3 guidance from Jersey Planning |  |
| Convert any regular bare patches to permanent grass or wildflower | Permanent vegetation builds SOC; prevents erosion and N₂O loss | RJA&HS; Farm Jersey agri-environment |  |
| Restore or maintain any traditional orchard | Veteran trees store large amounts of above-ground carbon | RJA&HS; National Trust for Jersey |  |
| Create or restore a small pond | Wetland carbon sequestration; also key habitat for agile frog and other priority species | Jersey Wildlife; Durrell; Birds on the Edge |  |
| Reduce tillage intensity | Less disturbance = less SOC oxidised; proven route to soil carbon build | RJA&HS agronomist; Farm Jersey |  |
| [Add your own action] |  |  |  |

# Section 6 – Your Carbon Reduction Actions

Now you know where your emissions come from, use this section to plan what you will do about them. Focus on the biggest sources first — for most Jersey smallholders that means diesel/oil/LPG and fertiliser use. Even one or two practical changes a year will add up.

📈 Jersey-specific funding and support for carbon reduction Government of Jersey Low Carbon Heating Incentive — up to £5,000 matched funding to switch to heat pump: gov.je/lowcarbonheating eco-active business network — free membership, carbon monitoring tools, energy advice: gov.je/ecoactive JEC Low Carbon Tariffs — contact JEC (01534 505460) for EV charging and heat pump tariffs; also ask about export rates for solar PV Farm Jersey / RJA&HS — agronomists who can advise on precision nutrition to reduce fertiliser use and N₂O Jersey Farmers’ Union — jfu.je; peer support and knowledge sharing on sustainable farming National Trust for Jersey — free trees and bank management advice for conservation-led landholders Birds on the Edge — birdsontheedge.org; agri-environment support and advice; can help identify Jersey-specific grant opportunities

## 6.1 Your Reduction Action Plan

| Action I Will Take | Emission Source Targeted | Estimated Saving (kgCO₂e/yr) | Cost / Investment | Target Date | Progress / Notes |
| --- | --- | --- | --- | --- | --- |
| Switch diesel irrigation pump to JEC electric pump | Diesel (pumping) | Litres saved × 2.56 | Medium capital; Gov incentive possible | Year 1–2 |  |
| Replace oil boiler with air-source heat pump | Heating oil | Litres saved × 2.54 | Gov incentive up to £5,000 | Year 1–2 |  |
| Switch van / 4WD to electric vehicle | Petrol / diesel (transport) | Litres saved × 2.3–2.56 | Capital; JEC EV tariff helps | Year 2–3 |  |
| Get a soil test and reduce N fertiliser rate | Soil N₂O + fertiliser manufacture | ~6.5 kgCO₂e per kg N reduced | Low (test cost ~£30–60) | Year 1 |  |
| Switch from synthetic N to compost / manure | Fertiliser manufacture | ~4.3 kgCO₂e per kg N replaced | Low; may reduce input cost | Year 1 |  |
| Install LED lighting in any polytunnel / store | JEC electricity | Small but visible | Low; pays back quickly | Year 1 |  |
| Plant [N] new trees this year | Long-term sequestration | ~0.4–2 tCO₂e/ha/yr as trees mature | Very low or free (NTJ) | Year 1 |  |
| Add cover crop between growing seasons | Soil N₂O + SOC | Moderate; builds over time | Seed cost only | Year 1 |  |
| Log all refrigerant top-ups; plan lower-GWP replacement | Refrigerants (if applicable) | Significant if leaks are occurring | Plan ahead for replacement | At next service |  |
| [Add your own action here] |  |  |  |  |  |
| [Add your own action here] |  |  |  |  |  |

## 6.2 Your Emission Reduction Targets

| Baseline year footprint (kgCO₂e) | [From Section 4 summary table] |
| --- | --- |
| Target for Year 2 (kgCO₂e) | [Set an achievable target: suggest −5 to −10%] |
| Target for Year 3 | [Continue the trend] |
| Target for Year 5 (2030 horizon) | [Aim for at least −30% vs baseline to contribute to Jersey’s 68% by 2030 national goal] |
| Your net zero target year | [YYYY — when you aim to reach net zero after sequestration and any offsets] |

# Section 7 – Annual Review & Sign-Off

Review this plan once a year — or sooner if you make a significant change to your holding (new livestock, new equipment, change of enterprise). The review doesn’t need to be long; the goal is to update your footprint figures, check progress on your actions, and set new targets.

| Review Date | Footprint This Year (kgCO₂e) | vs Baseline (%) | Sequestration (kgCO₂e) | Net Position (kgCO₂e) | Actions Completed | Main Challenges | Priority for Next Year |
| --- | --- | --- | --- | --- | --- | --- | --- |

## Sign-Off

| I confirm this plan represents an honest attempt to measure and reduce the greenhouse gas emissions from my holding, in support of Jersey’s net zero ambition. |  |
| --- | --- |
| Signed | [Signature] |
| Printed Name | [Name] |
| Date | [DD/MM/YYYY] |
| Next review | [DD/MM/YYYY] |

# Section 8 – Jersey Contacts & Tools

| Government of Jersey – Carbon Neutral Roadmap | gov.je/Environment/ClimateEmergency – Jersey’s legally binding climate targets |
| --- | --- |
| eco-active business network (free carbon tools) | gov.je/ecoactive – free energy monitoring spreadsheet and support for businesses |
| Gov of Jersey Low Carbon Heating Incentive | gov.je/lowcarbonheating – up to £5,000 matched funding to switch to heat pump |
| Jersey Electricity Company (JEC) | 01534 505460 ¦ jec.co.uk – energy, solar, EV tariffs, grid enquiries |
| JEC annual carbon intensity report | jec.co.uk/sustainability – current year emission factor for Scope 2 calculations |
| Farm Jersey (farming conferences, agronomy) | farmjersey@gmail.com ¦ jerseyfarmingconference.com |
| Royal Jersey Agricultural & Horticultural Society (RJA&HS) | 01534 866555 ¦ royaljersey.co.uk – agronomy, events, peer network |
| Jersey Farmers’ Union | jfu.je |
| National Trust for Jersey (trees, banks, habitats) | 01534 483193 ¦ nationaltrust.je |
| Birds on the Edge (agri-environment advice) | birdsontheedge.org – free advice on habitat management and carbon on-farm |
| Jersey Wildlife (agile frog, biodiversity) | jerseywildlife.org |
| LEAF Marque | leafuk.org – carbon plan requirements at annual audit |
| Red Tractor Assurance | redtractor.org.uk – environment section carbon requirements |
| Cool Farm Tool (free carbon calculator) | coolfarming.org – free, science-based, used by LEAF farms |
| AHDB Farm Carbon Calculator | ahdb.org.uk/carbon – free; good for livestock and arable |
| Farm Carbon Cutting Toolkit (sequestration) | farmcarbontoolkit.org.uk – practical guidance on measuring and building farm carbon sinks |
| DEFRA/BEIS GHG Conversion Factors | gov.uk/government/collections/government-conversion-factors-for-company-reporting – annual update |
| Farming Community Network (FCN) – wellbeing | 03000 111 999 ¦ fcn.org.uk – confidential support for farmers and families |

Every action counts — even the smallest holding can contribute to Jersey’s net zero journey.

Paris Agreement extended to Jersey, May 2022 • Carbon Neutral Roadmap: -68% by 2030, net zero by 2050