3D Printing Overview

We offer the some of the highest resolution and most accurate parts on the market

We aim to offer a next day delivery service where possible

We aim to supply a quotation within an hour of receiving an enquiry

We offer overnight delivery to all areas of the UK

We offer free delivery to the local area

We always aim to advise if there is an issue with the CAD or an area we can advise to adjust to produce the best parts possible

Tolerance for high resolution is +/- 0.15mm

We can run multiple materials overnight if a combination of materials or colours are required

Parts are supplied with a off machine finish or a bead blast finish for a uniform finish excludingt Ultra 10122 which is standard off machine finish or a polished/lacquered finish

Overview of available 3d print methods and materials

| SLA - Stereolithography |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Material | Colour | Material Mimic | High Resolution – 0.05mm | Standard Resolution – 0.1mm | High Resolution Build Envelope | Standard Resolution Build Envelope | Special Features |  |
| Xtreme | Grey | ABS/ PC | Yes | Yes | 450 x 450 x 450 | 500 x 500 x 500 | Suited to complex Assemblies |  |
| Xtreme White | White | ABS/ PC | Yes | Yes | 250 x 250 x 250 | 250 x 250 x 250 | Slightly tougher than extreme grey |  |
| Accura 25 | White | Polypropylene | Yes | Yes | 250 x 250 x 250 | 250 x 250 x 250 | For parts that need an extra bit of flex |  |
| Ultra 10122 | Clear | Polycarbonate | Yes | Yes | 250 x 250 x 250 | 250 x 250 x 250 | Can be post processed to a water clear finish |  |
| MPC 12 | Clear | ABS | No | Yes | N/A | 500 x 500 x 500 | Economical option for larger parts, can also be polished to a clear finish |  |
| Rapid White | White | ABS | Yes | Yes | 250 x 250 x 250 | 250 x 250 x 250 | High speed rapid prototyping resin for parts needing extra flexibility |  |
| Carbon - DLS |  |  |  |  |  |  |  |
| Material | Colour | Material Mimic | Extreme Resolution – 0.01mm | High Resolution – 0.05mm | Standard Resolution – 0.1mm | Build Envelope | Special Features |
| EPU40 | Black | Polyurethane | Yes | Yes | Yes | 189 x 118 x 326 mm | high-performance polyurethane elastomer that is a good choice for applications where high elasticity and tear resistance are needed |
| EPX 86FR | Black | Polyamides/Nylons | Yes | Yes | Yes | 189 x 118 x 326 mm | Flame-retardant, functional toughness, high strength, and long-term stability |
| EPX 82 | Black | Polycarbonate, Polypropylene | Yes | Yes | Yes | 189 x 118 x 326 mm | High-Strength, Long-Term Durability, and Functional Toughness |
| FPU 50 | Black | Polypropylene | Yes | Yes | Yes | 189 x 118 x 326 mm | Impact, abrasion and fatigue resistant semi-rigid material ideal choice for parts that must withstand repetitive stresses such as living hinges or friction fits. |
| MPU 100 | White | Medical-Grade ABS. | Yes | Yes | Yes | 189 x 118 x 326 mm | Offers a unique combination of mechanical strength, biocompatibility, and sterilizability. |
| RPU 70 | Black | ABS | Yes | Yes | Yes | 189 x 118 x 326 mm | Tough, rigid material that is a good choice for parts requiring strength, toughness, and moderate heat-resistance. |
| SIL 30 | Grey | Silicone Urethane Elastomer | Yes | Yes | Yes | 189 x 118 x 326 mm | soft, biocompatible, and tear resistant silicone urethane, ideal for skin Contact |

| FDM - Fused Deposition Modeling |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Material | Material Designation | Colour | Extreme Resolution | High Resolution | Standard Resolution | Build Envelope | Special Features |
| TPU 90A | Thermoplastic Polyurethane | Black - Variable | 0.08mm- 0.12mm | 0.14mm-0.16mm | 0.2mm-0.24mm | 350x320x325mm³/ 340x320x340mm³ | Flexible rubber like material with a 90A shore hardness give soft and bendable yet durable parts ideal for functional use or prototyping |
| ASA-CF | Acrylonitrile styrene acrylate | Grey - Variable | 0.08mm- 0.12mm | 0.14mm-0.16mm | 0.2mm-0.24mm | 350x320x325mm³/ 340x320x340mm³ | ASA reinforced with carbon fibre, making it stiff strong, heat resistant, uv resistant, ideal for outdoor or structural prints |
| PA6 | Nylon | Black/White/Natural | 0.08mm- 0.12mm | 0.14mm-0.16mm | 0.2mm-0.24mm | 350x320x325mm³/ 340x320x340mm³ | strong tough and wear resistant engineering plastic with high impact resistance and slight flexibility, material properties change overtime if in raw state |
| PETG | Polyethylene terephthalate glycol | Variable including translucent | 0.08mm- 0.12mm | 0.14mm-0.16mm | 0.2mm-0.24mm | 350x320x325mm³/ 340x320x340mm³ | Strong, slightly flexible with strong chemical resistant properties and moderate heat resistance. |
| ABS | Acrylonitrile butadiene styrene | White/Black - Variable | 0.08mm- 0.12mm | 0.14mm-0.16mm | 0.2mm-0.24mm | 350x320x325mm³/ 340x320x340mm³ | Strong, Rigid, moderate heat resistance, moderate impact strength, machinable |
| PPA-CF | Polyphthalamide | White - Variable | 0.08mm- 0.12mm | 0.14mm-0.16mm | 0.2mm-0.24mm | 350x320x325mm³/ 340x320x340mm³ | Nylon reinforced with carbon fibre, high strength, high heat resistance, lower moisture absorption compared to PA6 |
| PLA | Polylactic Acid | Variable | 0.08mm- 0.12mm | 0.14mm-0.16mm | 0.2mm-0.24mm | 350x320x325mm³/ 340x320x340mm³ | Rigid bio plastic, visual models only |

|  |  | DLP – Digital Light Proccessing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Material | Material Designation |  | Colour | Standard Resolution | High Resolution | Build Envelope | Special Features |
| Flexible X | Photopolymer |  | Black | 0.05mm | 0.025mm | 228.1x128.3x250 mm³ | High elongation, Excellent rebound, Good tear strength, Durable soft touch, highly flexible with a 55A shore hardness |