# PPWR Declaration of Conformity Pack

Editable template and practical evidence checklists

Use this pack to prepare a declaration for an identified packaging type after demonstrating conformity with the applicable requirements. The manufacturer remains responsible for packaging conformity; a supplier document or a completed template alone does not establish it.

## Contents

Pages 2–3  Blank Annex VIII declaration

Pages 4–5  Fictional completed example

Page 6  Field completion and release instructions

Page 7  Supplier evidence request checklist

Page 8  Change review and document control checklist

## Before you start

Identify the legal manufacturer and the packaging type. Keep the PPWR manufacturer role separate from the EPR producer role, which depends on the relevant market and supply chain.

Assess scope, exceptions and applicable dates before using the template. The PPWR generally applies from 12 August 2026, but individual requirements have different application dates.

Prepare the Annex VII technical file and complete conformity assessment. Record why each relevant requirement applies now, applies later or does not apply.

Use the fictional example only to understand the format. Its entities, specifications and evidence references are invented and cannot support a real declaration.

## Source and version

EcoComply working resource, version 1.0. Legal text checked on 13 September 2026. Basis: Regulation (EU) 2025/40, Articles 15–17, 38–39 and Annexes VII–VIII. Check subsequent amendments and applicable implementing measures when preparing a declaration.

https://eur-lex.europa.eu/eli/reg/2025/40/oj/eng

# EU declaration of conformity

No [declaration identification number]  |  Revision [number]

## 1  Unique identification of the packaging

[Unique packaging type identifier and revision]

## 2  Manufacturer and authorised representative

Manufacturer: [full legal name and postal address]
Manufacturer’s authorised representative, where applicable: [full legal name and address, or not applicable]

## 3  Sole responsibility

This declaration of conformity is issued under the sole responsibility of the manufacturer.

## 4  Object of the declaration

[Description identifying the packaging and allowing traceability: intended use, materials and components, dimensions, specification or drawing references, revision and batch identification. Identify the precise scope and any exclusions.]

## 5  Relevant Union harmonisation legislation

The object of the declaration referred to point 4 is in conformity with the relevant Union harmonisation legislation: [Regulation (EU) 2025/40 and references to other Union acts applied, including their publication references where appropriate].

# EU declaration of conformity continued

No [same declaration identification number]  |  Revision [same number]

## 6  Standards and specifications

[References, versions and relevant parts of harmonised standards, common specifications or other technical specifications actually used in relation to which conformity is declared. Do not list standards that were not applied.]

## 7  Notified body where applicable

[Where applicable: notified body name, address and number; description of intervention; certificate details, issue dates and, where appropriate, duration and conditions of validity. Otherwise record not applicable with the appropriate basis.]

## 8  Additional information

[Relevant additional information, for example technical file reference, scope clarification, applicability assessment reference and version. Keep the supporting evidence in the technical file.]

## Signature

Signed for and on behalf of: [legal manufacturer]
Place and date of issue: [place and date]
Name and function: [authorised signatory and function]
Signature: [signature]

Completion note: Replace every bracketed field and remove this note before issuing. This editable model follows Annex VIII; it does not demonstrate conformity by itself. Complete the conformity assessment and supporting technical documentation before signing. Confirm destination-language requirements.

Model source: Regulation (EU) 2025/40, Annex VIII.
https://eur-lex.europa.eu/eli/reg/2025/40/oj/eng

# EU declaration of conformity

ILLUSTRATIVE ONLY — NOT VALID FOR USE

No EXAMPLE-DOC-001  |  Revision 01

## 1  Unique identification of the packaging

EXAMPLE-BOX-001, revision A.

## 2  Manufacturer and authorised representative

Example Manufacturer Ltd (fictional entity)
1 Fictional Street, Example City, Germany (fictional address)
Manufacturer’s authorised representative: none in this example.

## 3  Sole responsibility

This declaration of conformity is issued under the sole responsibility of the manufacturer.

## 4  Object of the declaration

Single-use unprinted corrugated cardboard box, 200 × 150 × 100 mm internal dimensions, nominal mass 80 g, with integral closing flaps. Non-food-contact packaging for dry household goods. Board composition and tolerances: fictional specification EX-SPEC-001 rev A. Traceability: EXAMPLE-BOX-001 rev A and batch code on each box; drawing EX-DWG-001 rev A. No tape, label or insert is covered by this illustrative scope.

## 5  Relevant Union harmonisation legislation

The object of the declaration referred to point 4 is in conformity with the relevant Union harmonisation legislation: Regulation (EU) 2025/40, OJ L, 2025/40, 22.1.2025. No other Union act requiring an EU declaration of conformity is assumed for this fictional packaging scope.

# EU declaration of conformity continued

ILLUSTRATIVE ONLY — NOT VALID FOR USE

No EXAMPLE-DOC-001  |  Revision 01

## 6  Standards and specifications

Other technical specifications used: EX-SPEC-001 rev A (packaging specification) and EX-ASSESS-001 rev A (assessment methods and acceptance criteria), both fictional. No harmonised standard or common specification is claimed in this example. The hypothetical technical file records the solutions used to address applicable requirements; these references are not evidence of actual conformity.

## 7  Notified body where applicable

Not applicable. This fictional example uses the Annex VII internal production control procedure and assumes no other applicable Union act requiring notified body intervention.

## 8  Additional information

Illustrative technical file: EX-TF-001 rev A. Hypothetical applicability assessment: EX-APP-001 rev A, dated 13 September 2026. Food-contact use is excluded. Later requirements are not represented as already met. All entities, records and evidence references are fictional. This example demonstrates completion format only.

## Signature

Signed for and on behalf of: Example Manufacturer Ltd (fictional)
Place and date of issue: Example City, 13 September 2026 (illustrative)
Name and function: Example Signatory, Compliance Director (fictional)
Signature: NOT SIGNED — ILLUSTRATIVE ONLY

Model source: Regulation (EU) 2025/40, Annex VIII.
https://eur-lex.europa.eu/eli/reg/2025/40/oj/eng

# Field completion and release instructions

| Field | How to complete it |
| --- | --- |
| Number and field 1 | Use separate internal references for the declaration and packaging type. Neither is an EPR registration number. |
| 2 Responsible entity | Identify the legal manufacturer, not automatically the converter or EPR producer. Record any manufacturer’s AR and verify its written mandate. |
| 3 Responsibility | Retain the sole-responsibility statement. An AR does not take over the manufacturer’s responsibility for conformity. |
| 4 Traceable scope | Describe the packaging type and reference controlled specifications. Several SKUs may share a declaration only when the same conformity-relevant type is covered. |
| 5 Legislation | List applicable Union acts accurately. A packaged product and its packaging may need a coordinated declaration or dossier under Article 39(3). |
| 6 Methods | Use only standards or specifications actually applied. Keep their versions, partial application and alternative solutions in the technical file. |
| 7 Notified body | PPWR Annex VII uses internal production control. Complete notified body information only where applicable; do not invent a certificate or imply certification is routine. |
| 8 Additional information | Reference the technical file and applicability assessment. Avoid using this field as a substitute for evidence. |
| Signature | Verify the signatory’s authority, the named manufacturer, date, place and controlled document version. |

## Release checks

[ ] Applicable requirements assessed   [ ] Evidence supports the declared type
[ ] Technical file complete   [ ] Manufacturing controls in place
[ ] All placeholders removed   [ ] Required translations prepared
[ ] Authorised signature obtained   [ ] Controlled copy and evidence archived

Retain the declaration and technical documentation for 5 years from placement on the market for single-use packaging, or 10 years for reusable packaging. Maintain records that support the retention period for the units placed on the market. Sources: Article 15(2)–(4), Article 39 and Annex VII.

# Supplier evidence request checklist

Supplier [name]   Packaging type [ID]   Request owner [name]
Requested by [date]   Response due [date]   Specification revision [ID]

Request evidence relevant to the identified packaging and intended use. For each item, record received, missing or not applicable, with a reason. This is a practical request list, not a claim that every listed document is mandatory for every packaging type.

| Evidence to request | Scope to confirm | Status and reference |
| --- | --- | --- |
| Controlled specification | Materials, components, dimensions, tolerances, weights, drawings and intended use. | [Complete] |
| Composition information | Substrates, coatings, inks, adhesives and additives relevant to the assessment; quantities or concentrations where needed. | [Complete] |
| Substances evidence | Support for applicable Article 5 restrictions, including relevant heavy-metal limits and other applicable substance rules. | [Complete] |
| Food-contact PFAS evidence | Only where food-contact packaging is in scope: relevant assessment, supporting analyses and testing basis for Article 5 limits. | [Complete] |
| Tests and assessments | Reports tied to the supplied material/type, sample and date; methods, results, limitations and rationale for representativeness. | [Complete] |
| Other applicable requirements | Evidence for recyclability, minimisation, reusability or recycled content as relevant to the packaging and applicable date. Identify future-only items. | [Complete] |
| Supplier declarations | Issuer, packaging scope, applicable legislation, version and evidence references. Confirm whether a PPWR DoC or another declaration is being supplied. | [Complete] |
| Production and traceability | Batch identification, quality controls and process information needed to link supplied packaging to the assessed specification. | [Complete] |
| Change notification | Contact and agreed notification process for changes to material, composition, supplier site, design or supporting evidence. | [Complete] |

Close each gap with an owner and due date. A supplier specification can support the technical file but does not automatically replace the manufacturer’s assessment or declaration. Source: Article 16 and Annex VII.

# Change review and document control checklist

Packaging type [ID]   Current DoC [ID/version]   Change request [ID]
Review owner [name]   Review date [date]   Planned introduction [date]

| Change trigger | Review required |
| --- | --- |
| Materials or composition | Compare substrates, coatings, inks, adhesives, additives and substances evidence with the assessed version. |
| Dimensions or construction | Check whether mass, thickness, shape, closures, components or tolerances affect conformity or packaging-type scope. |
| Supplier or production process | Check equivalence of specification, process controls, traceability and supporting evidence. A supplier change alone does not decide the outcome. |
| Intended use or destination | Reassess food-contact status, reusable/single-use classification, use conditions and destination-language requirements. |
| New SKU or configuration | Confirm that the existing declaration covers the actual packaging type. Record the mapping; do not assume one DoC per SKU or per configuration. |
| Law or assessment method | Check application dates, amended requirements, revised standards/specifications and changed assessment methods. |
| Evidence or quality concern | Assess failed tests, complaints, inconsistent supplier records or evidence that may no longer represent production. |

## Record the decision

[ ] No conformity impact identified; retain the documented rationale.
[ ] Conformity could be affected; reassessment required before release.
[ ] Update technical file, declaration or scope after assessment.
[ ] Existing declaration does not cover the proposed packaging type.

Evidence reviewed [references]
Decision and rationale [explanation]
Actions, owners and due dates [details]
Approved by [name/function/date]   Effective version [ID]

Article 15(4) requires reassessment where conformity could be affected; Article 39(2) requires the declaration to be continuously updated. The checklist supports that review. A change does not automatically require a new declaration number, but the controlled document must accurately cover the packaging and applicable requirements.