# SECTION 07 42 13

## ARCHITECTURAL METAL WALL PANELS

22 Gauge ASTM A606 Type 4 Weathering Steel (Corten)

Manufactured by LCA Metals

## PART 1 – GENERAL

### 1.01 SUMMARY

A. Section Includes:
1. Fabrication and supply of single-skin architectural metal wall panels fabricated from ASTM A606 Type 4 weathering steel (Corten).
2. Custom panel profiles as indicated on drawings.
3. Associated trim components and accessories required for installation.

B. Related Sections:
1. 05 50 00 – Metal Fabrications
2. 07 60 00 – Flashing and Sheet Metal

### 1.02 REFERENCES

A. ASTM A606 – Standard Specification for Steel, Sheet and Strip, High-Strength, Low-Alloy, with Improved Atmospheric Corrosion Resistance.
B. ASTM A568 – General Requirements for Steel Sheet.

### 1.03 SUBMITTALS

A. Product Data: Submit manufacturer’s technical data sheets describing materials, fabrication methods, and installation requirements.

B. Shop Drawings: Submit detailed shop drawings indicating panel profiles, dimensions, joint conditions, fastening methods, trim details, and attachment interfaces.

C. Samples:
1. Submit one (1) custom panel profile sample produced as a 3D-printed prototype illustrating panel geometry, rib configuration, and intended visual form.
2. Submit one (1) 6" x 6" material sample of 22 gauge ASTM A606 Type 4 weathering steel.

D. Manufacturer Qualifications: LCA Metals shall demonstrate experience in fabrication of architectural metal panel systems.

## PART 2 – PRODUCTS

### 2.01 MANUFACTURER

LCA Metals
190 Bissell Pl
San Jacinto, CA 92582
Contact: Ben Haas
Email: sales@metallicpanels.com

Substitutions: Not permitted without written approval.

### 2.02 MATERIALS

A. Base Material:
1. ASTM A606 Type 4 weathering steel (Corten).
2. Thickness: 22 gauge (minimum nominal thickness 0.0299").
3. Finish: Uncoated, natural mill finish intended to develop a stable oxide patina through atmospheric exposure.

B. Panel Fabrication:
1. Single-skin architectural wall panels.
2. Profiles: Custom as indicated on architectural drawings.
3. Panels fabricated to specified lengths to minimize field seams.
4. Edges formed, hemmed, or stiffened as required for rigidity and safety.

C. Fasteners:
1. Corrosion-resistant fasteners compatible with weathering steel.
2. Stainless steel or weathering steel-compatible fasteners.
3. Exposed or concealed fastening as indicated on drawings.

D. Accessories:
1. Matching weathering steel trim and flashing components.
2. Closure strips and sealants compatible with weathering steel.

### 2.03 WEATHERING CHARACTERISTICS

A. Weathering steel panels are supplied uncoated and will develop a natural protective oxide layer (patina) when exposed to alternating wet and dry atmospheric conditions.

B. Initial Weathering Phase (0–3 Months):
Panels will begin oxidation, exhibiting surface rust coloration ranging from orange to reddish-brown. During this phase, runoff staining may occur.

C. Intermediate Phase (3–12 Months):
Patina continues to deepen in tone and becomes more uniform. Surface runoff decreases as oxidation stabilizes.

D. Stabilization Phase (12–36 Months):
A dense, stable oxide layer develops, significantly slowing further corrosion. Final coloration will vary depending on climate, exposure orientation, and environmental conditions.

E. Proper detailing shall allow for drainage and drying to ensure uniform patina development and minimize staining of adjacent materials.

## PART 3 – EXECUTION

### 3.01 EXAMINATION

A. Verify substrates are plumb, level, and suitable to receive metal panels.
B. Do not begin installation until unsatisfactory conditions have been corrected.

### 3.02 INSTALLATION

A. Install panels in accordance with LCA Metals shop drawings and project specifications.
B. Allow for thermal expansion and contraction of weathering steel.
C. Align panels true to line and level.
D. Avoid direct contact between weathering steel and dissimilar metals that may cause galvanic corrosion.

### 3.03 PROTECTION

A. Protect installed panels from damage during construction.
B. Remove debris and construction contaminants from panel surfaces in accordance with manufacturer recommendations.

END OF SECTION