Ref: XXXX

Version   Date

Safety Data Sheet

HYRA-STIK EVO

Adhesive

Bitumen Waterproofing

# Section 1: Identification of the substance/mixture and of the company/undertaking

### Product identifier

Product name	HYRA-STIK EVO

Product number	HYRA 001

### Relevant identified uses of the substance or mixture and uses advised against Identified uses	Adhesive.

Uses advised against	No specific uses advised against are identified.

### 1.3.	Details of the supplier of the safety data sheet

Supplier	Address

Tel:

Email:

  - Emergency telephone	Company name and emergency telephone number

In the event of a medical enquiry involving this product, members of the public should contact:

NHS 111

a doctor or

a local hospital accident and emergency department.

The NPIS (National Poisons Information Service) helpline is available for enquiries from medical professionals only.

Tel: 0344 892 0111

# Section 2: Hazards identification

### Classification of the substance or mixture Classification (EC 1272/2008)

Physical hazards	Not Classified

Health hazards	Skin Irrit. 2 - H315 Eye Irrit. 2 - H319 Resp. Sens.

1 - H334 Skin Sens. 1 - H317 Carc. 2 - H351 STOT SE

3 - H335 STOT RE 2 - H373

Environmental hazards	Not Classified

Human health	May cause sensitisation by inhalation.

Physicochemical	Vapours are heavier than air and may travel along the floor and accumulate in the bottom of containers.

### Label elements

Pictogram

Signal word	Danger

Hazard statements	H315 Causes skin irritation.

H317 May cause an allergic skin reaction. H319 Causes serious eye irritation.

H334 May cause allergy or asthma symptoms or breathing difficulties if inhaled.

H335 May cause respiratory irritation. H351 Suspected of causing cancer.

H373 May cause damage to organs through prolonged or repeated exposure.

Precautionary statements	P260 Do not breathe vapour/ spray.

P280 Wear protective gloves/ protective clothing/ eye protection/ face protection.

P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P308+P313 IF exposed or concerned: Get medical advice/ attention.

P501 Dispose of contents/ container in accordance with national regulations.

Supplemental label information	EUH204 Contains isocyanates. May produce an allergic reaction

RCH004a Persons already sensitised to diisocyanates may develop allergic reactions when using this product.

RCH004b Persons suffering from asthma, eczema or skin problems should avoid contact, including dermal contact, with this product. RCH004c This product should not be used under conditions of poor ventilation unless a protective mask with an appropriate gas filter (i.e. type A1 according to standard EN 14387) is used.

Contains	DIPHENYLMETHANE-4,4’-DI-ISOCYANATE

### Supplementary precautionary

statements	P201 Obtain special instructions before use.

P202 Do not handle until all safety precautions have been read and understood.

P261 Avoid breathing vapour/ spray.

P264 Wash contaminated skin thoroughly after handling. P271 Use only outdoors or in a well-ventilated area.

P272 Contaminated work clothing should not be allowed out of the workplace.

P284 [In case of inadequate ventilation] wear respiratory protection. P302+P352 IF ON SKIN: Wash with plenty of water.

P304+P340 IF INHALED: Remove person to fresh air and keep comfortable for breathing.

P312 Call a POISON CENTRE/ doctor if you feel unwell. P314 Get medical advice/ attention if you feel unwell.

P321 Specific treatment (see medical advice on this label). P332+P313 If skin irritation occurs: Get medical advice/ attention.

P333+P313 If skin irritation or rash occurs: Get medical advice/ attention.

P337+P313 If eye irritation persists: Get medical advice/ attention. P342+P311 If experiencing respiratory symptoms: Call a POISON CENTRE/ doctor.

P362+P364 Remove contaminated clothing, and wash it before re-use.

P403+P233 Store in a well-ventilated place. Keep container tightly closed.

P405 Store locked up.

### Other hazards

This product does not contain any substances classified as PBT or vPvB.

# Section 3: Composition/information on ingredients

### 3.2	Mixtures

| DIPHENYLMETHANE-4,4’-DI-ISOCYANATE 44% CAS number: 101-68-8 EC number: 202-966-0 REACH registration number: 01- 2119457014-47-0000 |  |
| --- | --- |
| Classification Acute Tox. 4 - H332 Skin Irrit. 2 - H315 Eye Irrit. 2 - H319 Resp. Sens. 1 - H334 Skin Sens. 1 - H317 Carc. 2 - H351 STOT SE 3 - H335 STOT RE 2 - H373 |  |

| 2,2’DIMORPHOLINYLDIETHYL ETHER <1% CAS number 6425-39-4 EC number 229-194-7 REACH registration number: 01- 2119969278-20-0000 |  |
| --- | --- |
| Classification Eye Irrit. 2 - H319 |  |

| BENZOYL CHLORIDE <1% CAS number 98-88-4 EC number 202-710-8 REACH registration number: 01- 2119487138-29-0002 |  |
| --- | --- |
| Classification Acute Tox. 4 - H302 Acute Tox. 3 - H311 Acute Tox. 2 - H330 Skin Corr. 1B - H314 Skin Sens. 1 - H317 |  |

| PHOSPHORIC ACID ...% <1% CAS number: 7664-38-2 EC number: 231-633-2 REACH registration number: 01- 2119485924-24-0070 |  |
| --- | --- |
| Classification Met. Corr. 1 - H290 Acute Tox. 4 - H302 Skin Corr. 1B - H314 |  |

The full text for all hazard statements is displayed in Section 16.

# Section 4: First aid measures

### Description of first aid measures

General information	Remove affected person from source of contamination.

Inhalation	Move affected person to fresh air at once. Get medical attention if any discomfort continues.

Ingestion	DO NOT induce vomiting. Get medical attention immediately.

Skin contact	Remove contaminated clothing immediately and wash skin with soap and water. Get medical attention if any discomfort continues.

Eye contact	Rinse immediately with plenty of water. Remove any contact lenses and open eyelids wide apart. Continue to rinse for at least 15 minutes. Get medical attention if irritation persists after washing. Show this Safety Data Sheet to the medical personnel.

Protection of first aiders	First aid personnel should wear appropriate protective equipment

during any rescue.

### Most important symptoms and effects, both acute and delayed

General information	The severity of the symptoms described will vary dependent on the

concentration and the length of exposure.

Inhalation	Irritation of nose, throat and airway. Coughing, chest tightness, feeling of chest pressure.

Ingestion	May cause discomfort if swallowed.

Skin contact	Prolonged skin contact may cause redness and irritation.

Eye contact	Severe irritation, burning and tearing.

### Indication of any immediate medical attention and special treatment needed

Notes for the doctor	No specific recommendations. If in doubt, get medical

attention promptly.

Specific treatments	Treat symptomatically.

# Section 5: Firefighting measures

### Extinguishing media

Suitable extinguishing media	Extinguish with foam, carbon dioxide, dry powder or water fog.

Unsuitable extinguishing media	Do not use water jet as an extinguisher, as this will spread the fire.

### Special hazards arising from the substance or mixture

Specific hazards	The product is non-combustible. Irritating gases or vapours. Not known.

Hazardous combustion products	Thermal decomposition or combustion may liberate carbon oxides

and other toxic gases or vapours. Oxides of carbon. Oxides of nitrogen.

### Advice for firefighters

Protective actions during firefighting  Containers close to fire should be removed or cooled with water.

Do not allow water to contact any leaked material.

### Special protective equipment

for firefighters	Wear chemical protective suit. Wear positive-pressure self-contained breathing apparatus (SCBA) and appropriate protective clothing.

# Section 6: Accidental release measures

### Personal precautions, protective equipment and emergency procedures

Personal precautions	Wear protective clothing as described in Section 8 of this safety

data sheet.

### Environmental precautions

Environmental precautions	Do not discharge into drains or watercourses or onto the ground.

### Methods and material for containment and cleaning up

Methods for cleaning up	Absorb spillage with non-combustible, absorbent material.

Absorb spillage with non-combustible, absorbent material. Collect and place in suitable waste disposal containers and seal securely. Provide adequate ventilation. Contain spillage with sand,

earth or other suitable non-combustible material. Avoid the spillage or run-off entering drains, sewers or watercourses.

### Reference to other sections

Reference to other sections	Wear protective clothing as described in Section 8 of this safety

data sheet.

# Section 7: Handling and storage

### 7.1	Precautions for safe handling

Usage precautions	Avoid inhalation of vapours and spray/mists. Avoid contact with

skin and eyes. Do not use in confined spaces without adequate ventilation and/or respirator. Spraying is permitted only in closed systems, spray cabinets or spray boxes with adequate ventilation.

### Advice on general

occupational hygiene	Wash promptly with soap and water if skin becomes contaminated.

Preventive industrial medical examinations should be carried out.

### Conditions for safe storage, including any incompatibilities

Storage precautions	Store in closed original container at temperatures between 5°C

and 25°C.

Storage class	Chemical storage.

### Specific end use(s)

Specific end use(s)	The identified uses for this product are detailed in Section 1.2.

# Section 8: Exposure Controls/personal protection

Additional information about design of technical facilities: No further data; see item 7.

  - Control parameters Occupational exposure limits

## DIPHENYLMETHANE-4,4’-DI-ISOCYANATE

Long-term exposure limit (8-hour TWA): WEL 0.02 mg/m3(Sen) Short-term exposure limit (15-minute): WEL 0.07 mg/m3(Sen)

## PHOSPHORIC ACID ...%

Long-term exposure limit (8-hour TWA): WEL 1 mg/m³ Short-term exposure limit (15-minute): WEL 2 mg/m³ WEL = Workplace Exposure Limit

Ingredient comments	WEL = Workplace Exposure Limits

## DIPHENYLMETHANE-4,4’-DI-ISOCYANATE (CAS: 101-68-8)

DNEL	Workers - Inhalation; Short term systemic effects: 0.1 mg/m³ Workers - Dermal; Short term local effects: 28.7 mg/cm² Workers - Inhalation; Short term local effects: 0.1 mg/m³ Workers - Inhalation; Long term systemic effects: 0.05 mg/m³ Workers - Inhalation; Long term local effects: 0.05 mg/m³

Consumer - Dermal; Short term systemic effects: 25 mg/kg bw/day Workers - Dermal; Short term systemic effects: 50 mg/kg bw/day Consumer - Oral; Short term systemic effects: 20 mg/kg bw/day Consumer - Dermal; Short term local effects: 17.2 mg/cm² Consumer - Inhalation; Short term local effects: 0.05 mg/m³ Consumer - Inhalation; Long term systemic effects: 0.025 mg/m³ Consumer - Inhalation; Long term local effects: 0.025 mg/m³ Consumer - Inhalation; Short term systemic effects: 0.05 mg/m³

PNEC	- Marine water; 0.1 mg/l

- STP; 1 mg/l

- Fresh water; 1 mg/l

- Soil; 1 mg/kg

## 2,2’DIMORPHOLINYLDIETHYL ETHER (CAS: 6425-39-4)

DNEL	Workers - Inhalation; Long term systemic effects: 7.28 mg/m³ Workers - Dermal; Long term systemic effects: 1 mg/kg bw/day Consumer - Inhalation; Long term systemic effects: 1.8 mg/m³ Consumer - Dermal; Long term systemic effects: 0.5 mg/kg bw/day Consumer - Oral; Long term systemic effects: 0.5 mg/kg bw/day

PNEC	- Fresh water; 0.1 mg/l

- Marine water; 0.01 mg/l

- Intermittent release; 1 mg/l

- Sediment (Freshwater); 8.2 mg/kg

- Sediment (Marinewater); 0.82 mg/kg

- STP; 100 mg/l

- Soil; 1.58 mg/kg

### Exposure controls Protective equipment

Appropriate engineering controls	Provide adequate ventilation. Avoid inhalation of vapours.

Observe any occupational exposure limits for the product or ingredients.

Eye/face protection	Wear chemical splash goggles.

Hand protection	It is recommended that gloves are made of the following material: Nitrile rubber. It should be noted that liquid may penetrate the gloves. Frequent changes are recommended. For exposure up to 8 hours, wear gloves made of the following material: Nitrile rubber.

Other skin and body protection	Wear suitable protective clothing as protection against splashing

or contamination. Wear apron or protective clothing in case of contact.

Hygiene measures	Use engineering controls to reduce air contamination to permissible

exposure level. Wash hands after handling. When using do not eat, drink or smoke.

Respiratory protection	If ventilation is inadequate, suitable respiratory protection must be

worn. If ventilation is inadequate, suitable respiratory protection must be worn. Wear a respirator fitted with the following cartridge: Combination filter, type A2/P3.

Environmental exposure controls	Keep container tightly sealed when not in use.

# Section 9: Physical and Chemical Properties

### Information on basic physical and chemical properties Appearance	Coloured liquid.

Colour	Various colours.

Odour	Musty (mouldy).

Odour threshold	Not available.

pH	Estimated value. pH (concentrated solution): 7-8

Melting point	<10°C

Initial boiling point and range	330°C @ mbar

Flash point	>200°C CC (Closed cup).

### Evaporation rate	slow

Evaporation factor	Not available.

Flammability (solid, gas)	Not available.

Other flammability	Not available.

Vapour pressure	0.01 Pa @ °C

### Vapour density	8.5

Relative density	1.12 @ 20°C

Bulk density	Not available.

Solubility(ies)	Insoluble in water. Hardens in contact with water.

Partition coefficient	Not available.

Auto-ignition temperature	>600°C Decomposition Temperature	Not available. Viscosity	>2000 cP @ 25°C

Explosive properties	Not available.

### Explosive under the influence

of a flame	Not considered to be explosive.

Oxidising properties	Not available.

Comments	Information given is applicable to the product as supplied.

### 9.2.	Other information

Other information	No information required.

Refractive index	Not available.

Particle size	Not available.

Molecular weight	Not available.

Volatility	Not available.

Saturation concentration	Not available.

Critical temperature	Not available.

Volatile organic compound	No information available.

# Section 10: Stability and reactivity

| 10.1 | Reactivity |  |
| --- | --- | --- |
|  | Reactivity | The product will harden into a solid mass in contact with water and moisture. |
| 10.2. | Chemical stability |  |
|  | Stability | Stable at normal ambient temperatures and when used as recommended. |
| 10.3. | Possibility of hazardous reactions |  |
|  | Possibility of hazardous reactions | Not applicable. May polymerise. |
| 10.4. | Conditions to avoid |  |
|  | Conditions to avoid | Avoid contact with water. |
| 10.5. | Incompatible materials |  |
|  | Materials to avoid | Strong oxidising agents. |

### 10.6.	Hazardous decomposition products

Hazardous decomposition products Thermal decomposition or combustion may liberate carbon oxides

and other toxic gases or vapours. Oxides of carbon. Oxides of nitrogen.

# SECTION 11: Toxicological information

### 11.1.	Information on toxicological effects

Toxicological effects	No information available.

Other health effects	There is no evidence that the product can cause cancer.

Acute toxicity - inhalation

### ATE inhalation (vapours mg/l)	25.0

Skin corrosion/irritation

Animal data	Irritating.

### Serious eye damage/irritation

Serious eye damage/irritation	Moderately irritating.

### Respiratory sensitisation

Respiratory sensitisation	Sensitising.

### Skin sensitisation

Skin sensitisation	Not determined.

### Carcinogenicity

Carcinogenicity	Suspected carcinogen based on limited evidence.

### Target organ for

carcinogenicity	No specific target organs known.

### Reproductive toxicity

Reproductive toxicity - fertility	Not available.

Reproductive toxicity - development  This substance has no evidence of toxicity to reproduction.

### Specific target organ toxicity - repeated exposure

STOT - repeated exposure	Morphological changes that are potentially reversible but provide

clear evidence of marked organ dysfunction.

### Aspiration hazard

Aspiration hazard	Not anticipated to present an aspiration hazard, based on chemical

structure.

General information	No specific health hazards known.

Inhalation	Irritating to respiratory system. May cause sensitisation by inhalation.

Ingestion	May cause stomach pain or vomiting. Skin contact Irritating to skin.	May cause sensitisation by skin contact. Eye contact	Irritation of eyes and mucous membranes.

Acute and chronic health hazards	May cause sensitisation by skin contact. The product contains small

quantities of isocyanate. May cause respiratory allergy. May cause respiratory system irritation. May cause respiratory system irritation. Frequent inhalation of vapours may cause respiratory allergy.

Route of entry	Inhalation Skin and/or eye contact

Medical symptoms	Irritation of eyes and mucous membranes. Coughing, chest tightness, feeling of chest pressure.

Medical considerations	Chronic respiratory and obstructive airway diseases.

### Toxicological information on ingredients. DIPHENYLMETHANE-4,4’-DI-ISOCYANATE

Acute toxicity - oral

Acute toxicity oral (LD50 mg/kg)	10,000.0

Species	Rat

ATE oral (mg/kg)	10,000.0

### Acute toxicity - dermal

Acute toxicity dermal (LD50 mg/kg)	9,400.0

Species	Rabbit

ATE dermal (mg/kg)	9,400.0

### Acute toxicity - inhalation Acute toxicity inhalation

(LC50 vapours mg/l)	0.31

Species	Rat

### ATE inhalation (vapours mg/l)	11.0

Carcinogenicity

IARC carcinogenicity	IARC Group 3 Not classifiable as to its carcinogenicity to humans.

## 2,2’DIMORPHOLINYLDIETHYL ETHER

### Acute toxicity - oral

Acute toxicity oral (LD50 mg/kg)	2,025.0

Species	Rat

Notes (oral LD50)	No information available.

### Acute toxicity - dermal

Acute toxicity dermal (LD50 mg/kg)	3,038.0

Species	Rabbit

Notes (dermal LD50)	No information available.

### Acute toxicity - inhalation

Notes (inhalation LC50)	No information available.

### Skin corrosion/irritation

Skin corrosion/irritation	No information available.

### Serious eye damage/irritation

Serious eye damage/irritation	No information available.

### Respiratory sensitisation

Respiratory sensitisation	No information available.

### Skin sensitisation

Skin sensitisation	No information available.

### Carcinogenicity

IARC carcinogenicity	No component of this product present at levels greater than or

equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Inhalation	May be harmful if inhaled. Spray/mists may cause respiratory tract irritation.

Ingestion	May be harmful if swallowed.

Skin contact	May be absorbed through the skin. May be harmful in contact with skin. May cause skin irritation.

Eye contact	May cause eye irritation.

## BENZOYL CHLORIDE

### Acute toxicity - oral

Acute toxicity oral (LD50mg/kg)	1,900.0

Species	Rat

ATE oral (mg/kg)	1,900.0

### Acute toxicity - dermal

Acute toxicity dermal (LD50mg/kg)	790.0

Species	Rat

ATE dermal (mg/kg)	790.0

### Acute toxicity - inhalation Acute toxicity inhalation

(LC50 vapours mg/l)	1.45

Species	Rat

ATE inhalation (vapours mg/l)	1.45

Carcinogenicity

IARC carcinogenicity	IARC Group 2A Probably carcinogenic to humans.

## PHOSPHORIC ACID	%

### Acute toxicity - oral

ATE oral (mg/kg)	500.0

# SECTION 12: Ecological Information

Ecotoxicity	The product is not expected to be hazardous to the environment.

### Toxicity

Acute toxicity - fish	LC50, 96 hours: > 1000 mg/l, Freshwater fish

Acute toxicity - aquatic invertebrates EC50, 48 hours: >500 mg/l, Daphnia magna

Acute toxicity - aquatic plants	EC50, 72 hours: ~ 1640 mg/l, Scenedesmus subspicatus

Ecological information on ingredients.

## DIPHENYLMETHANE-4,4’-DI-ISOCYANATE

Acute toxicity - fish	LC50, 96 hours: >1000 mg/l, Marinewater fish Acute toxicity - aquatic invertebrates EC50, 24 hours: >1000 mg/l, Daphnia magna Chronic toxicity - aquatic

invertebrates	NOEC, 21 days: >10 mg/l, Daphnia magna

## 2,2’DIMORPHOLINYLDIETHYL ETHER

Acute toxicity - fish	LC50, 96 hours: 2150 mg/l,

Acute toxicity - aquatic invertebrates EC50, 48 hours: >100 mg/l, Daphnia magna

Acute toxicity - microorganisms	EC50, 3 hours: >1000 mg/l, Bacteria

### Persistence and degradability

Persistence and degradability	The product is not readily biodegradable.

Stability (hydrolysis)	Reacts with water.

Biological oxygen demand	< 10 g O2/g substance

### Bioaccumulative potential

Bioaccumulative potential	The product does not contain any substances expected to be

bioaccumulating.

Partition coefficient	Not available.

Ecological information on ingredients.

## DIPHENYLMETHANE-4,4’-DI-ISOCYANATE

Partition coefficient	log Pow: 4.51

### Mobility in soil

Mobility	The product is non-volatile.

### Results of PBT and vPvB assessment

Results of PBT and vPvB assessment This product does not contain any substances classified as PBT or vPvB.

### Other adverse effects

Other adverse effects	None known.

# SECTION 13: Disposal considerations

### Waste treatment methods

General information	Waste should be treated as controlled waste. Dispose of waste to

licensed waste disposal site in accordance with the requirements of the local Waste Disposal Authority.

Disposal methods	Dispose of waste to licensed waste disposal site in accordance with

the requirements of the local Waste Disposal Authority.

# SECTION 14: Transport information

General	The product is not covered by international regulations on the transport of dangerous goods (IMDG, IATA, ADR/RID).

### UN number

Not applicable.

### UN proper shipping name

Not applicable.

### Transport hazard class(es)

No transport warning sign required.

### Transport labels

No transport warning sign required.

### Packing group

Not applicable.

### Environmental hazards

Environmentally hazardous substance/marine pollutant No.

### Special precautions for user

Not applicable.

### Transport in bulk according to Annex II of MARPOL and the IBC Code Transport in bulk according to

Annex II of MARPOL 73/78

and the IBC Code	Not applicable.

# SECTION 15: Regulatory information

  - Safety, health and environmental regulations/legislation specific for the substance or mixture National regulations	Health and Safety at Work etc. Act 1974 (as amended).

The Control of Substances Hazardous to Health Regulations 2002 (SI 2002 No. 2677) (as amended).

The Chemicals (Hazard Information and Packaging for Supply) Regulations 2009 (SI 2009 No. 716).

Control of Substances Hazardous to Health Regulations 2002 (as amended).

EU legislation	Commission Directive 91/322/EEC of 29 May 1991 on establishing indicative limit values by implementing Council Directive 80/1107/ EEC on the protection of workers from the risks related to exposure to chemical, physical and biological agents at work. Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH) (as amended).

### Chemical safety assessment

No chemical safety assessment has been carried out.

# SECTION 16: Other information

Hazard statements in full	H290 May be corrosive to metals.

H302 Harmful if swallowed. H311 Toxic in contact with skin.

H314 Causes severe skin burns and eye damage. H315 Causes skin irritation.

H317 May cause an allergic skin reaction. H319 Causes serious eye irritation.

H330 Fatal if inhaled. H332 Harmful if inhaled.

H334 May cause allergy or asthma symptoms or breathing difficulties if inhaled.

H335 May cause respiratory irritation. H351 Suspected of causing cancer.

H373 May cause damage to organs through prolonged or repeated exposure.

Store Between	5oc - 25oc

### Contains SVHC	NO

This information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process. Such information is, to the best of the company’s knowledge and belief, accurate and reliable as of the date indicated. However, no warranty, guarantee or representation is made to its accuracy, reliability or completeness. It is the user’s responsibility to satisfy himself as to the suitability of such information for his own particular use.

The data contained in this SDS has been supplied as required by the EC REACH Regulation No. 1907/2006 and the EC Regulation No. 1272/2008 on classification, labelling and packaging of substances and mixtures (CLP) for the purpose of protecting the health and safety of industrial and commercial users who are deemed capable of

understanding and acting on the information provided. Please ensure that it is passed to the appropriate person(s) in your company who are capable of acting on the information.

Disclaimer:

XXXX reserves the right to modify and update this data at any time without prior notice. Only the latest version of this document is valid, available for download at XXXX. Once downloaded, documents are uncontrolled. Users should always confirm they are referring to the latest version prior to use. Further assistance is available from XXXX,

Email: XXXX telephone: XXXX

The intended use of this product should be verified with XXXX prior to adoption to ensure its suitability and compliance with specifications, project requirements, industry regulations, legislation, good practice, installation techniques and all other relevant guidance. The user should ensure all necessary precautions are taken during handling, storage, installation and disposal of the product, and all regulations to guarantee safety of people and the environment are observed. XXXX accepts no liability for non-compliant use of this product.