SPECIFICATION

Entrada Equinox Canopy NZ

Specification for the supply and installation of the Entrada Equinox cantilevered aluminium canopy system. Written in NZ Masterspec format for inclusion in the project specification.

| Project | [Project name] |
| --- | --- |
| Specifier | [Name and company] |
| Date | [DD/MM/YYYY] |

# 1	GENERAL

This section relates to the supply and installation of the Entrada Equinox Canopy system. Entrada supplies the Equinox as a pre-finished, structurally calculated kitset, installed by an installer approved by Entrada.

## 1.1	ABBREVIATIONS AND DEFINITIONS

Refer to the general section 1232 INTERPRETATION & DEFINITIONS for abbreviations and definitions used throughout the specification.

DOCUMENTS

## 1.2	DOCUMENTS

Refer to the general section 1233 REFERENCED DOCUMENTS. The following documents are specifically referred to in this section:

| AS/NZS 1163 | Cold-formed structural steel hollow sections |
| --- | --- |
| AS/NZS 1594 | Hot-rolled steel flat products |
| AS/NZS 1664.1 | Aluminium structures - Limit state design |
| AS/NZS 1734 | Aluminium and aluminium alloys - Flat sheets, coiled sheet and plate |
| AS/NZS 1866 | Aluminium and aluminium alloys - Extruded rod, bar, solid and hollow shapes |
| NZS 3404.1 | Steel structures Standard |
| NZS 3404.2 | Commentary to the Steel structures Standard |
| AS/NZS 3678 | Structural steel - Hot-rolled plates, floorplates and slabs |
| AS/NZS 4600 | Cold-formed steel structures |

## 1.3	MANUFACTURER/SUPPLIER DOCUMENTS

Manufacturer and supplier documents relating to this part of the work:

| Entrada | Equinox Canopy Technical Data Sheet |
| --- | --- |
| Entrada | Care & Maintenance Manual (ENT-CM) |

Manufacturer/supplier contact details

| Company | Entrada (a division of Insol Ltd) |
| --- | --- |
| Address | 7 Waokauri Place, Auckland |
| Web | entradacanopies.com |
| Email | enquiries@entradacanopies.com |
| Telephone | 09 276 9735 |

## 1.4	SPECIFIC DESIGN DOCUMENTS

Specific design documents relating to this part of the work:

| Documents | [List documents] |
| --- | --- |
| Refer to | [Drawing or document reference] |

WARRANTIES

## 1.5	WARRANTY - MANUFACTURER/SUPPLIER

Provide a material manufacturer/supplier warranty:

| [~] years | For the Entrada Equinox Canopy system |
| --- | --- |
| [~] years | For powder coat finish durability and colour |

Provide this warranty on the manufacturer/supplier standard form (if not available then use the standard form in the general section 1237WA WARRANTY AGREEMENT). Commence the warranty from the date of practical completion of the contract works.

Refer to the general section 1237 WARRANTIES for additional requirements.

## 1.6	WARRANTY - INSTALLER/APPLICATOR

Provide an installer/applicator warranty:

| [~] years | For installation |
| --- | --- |

Provide this warranty on the installer/applicator standard form (if not available then use the standard form in the general section 1237WA WARRANTY AGREEMENT). Commence the warranty from the date of practical completion of the contract works.

Refer to the general section 1237 WARRANTIES for additional requirements.

REQUIREMENTS

## 1.7	QUALIFICATIONS - MANUFACTURER/SUPPLIER REQUIREMENTS

Installers to be approved by Entrada. Refer to 1270 CONSTRUCTION for additional requirements relating to qualifications.

## 1.8	NO SUBSTITUTIONS

Substitutions are not permitted to any of the specified systems, components and associated products listed in this section.

## 1.9	SHOP DRAWINGS

Refer to the general section 1235 SHOP DRAWINGS for the requirements for submission and review and the provision of final shop drawings.

Provide shop drawings to show the general arrangement including, but not limited to:

- Overall canopy dimensions

- Wall bracket and fixing locations

- Flashing details

- Sealants and air seals

- Gutter and downpipe dropper location

- Non-standard fixing details including brackets

- Sequence/methodology of installation

If requested provide the following additional information:

- Provision for the exclusion and/or drainage of moisture

- Provision for adjustment of fixings to ensure true alignment of the canopy

- Provision for thermal movement

- Provision for seismic movement and movement under wind loads

- Information of Professional Indemnity Insurance held by the person providing the calculations and shop drawings

Submit shop drawings for review to [Name].

[~] working days (at least) before fabrication is planned to commence, provide shop drawings for review. Complete shop drawing review before commencing fabrication.

## 1.10	INFORMATION FOR OPERATION AND MAINTENANCE

Refer to the general section 1239 OPERATION & MAINTENANCE for provision of operation and maintenance information as electronic PDF format documents:

- Entrada Care & Maintenance Manual (ENT-CM)

Provide this information prior to practical completion.

## 1.11	INFORMATION REQUIRED FOR CODE COMPLIANCE

Provide the following compliance documentation:

- Producer Statement - Construction (PS3) from the installer/applicator

- Other information required by the BCA in the Building Consent Approval documents

# 2	PRODUCTS

MATERIALS

## 2.1	ALUMINIUM

| Sheet | Grade 5005 |
| --- | --- |
| Extrusion | Grade 6060 |

Aluminium structures to AS/NZS 1664.1. Aluminium alloys (solid and hollow shapes) to AS/NZS 1866. Aluminium sheets and plates to AS/NZS 1734.

## 2.2	STEEL

Steel structures to NZS 3404.1 and NZS 3404.2. Cold-formed steel structures to AS/NZS 4600, cold-formed steel sections to AS/NZS 1163. Hot-rolled steel plates to AS/NZS 3678, hot-rolled steel flats to AS/NZS 1594.

CANOPY

## 2.3	ENTRADA EQUINOX CANOPY SYSTEM

Cantilevered canopy system consisting of:

- Aluminium canopy tray, powder coat finish

- Fixings and fixing components

- Gutter

- Downpipe dropper

ACCESSORIES

## 2.4	SEALANT

Sikaflex®-11 FC, a one-component, gun-grade adhesive and sealing compound of permanent elasticity based on a moisture-cured polyurethane.

# 3	EXECUTION

CONDITIONS

## 3.1	DELIVERY, STORAGE AND HANDLING OF PRODUCTS

Refer to 1270 CONSTRUCTION for requirements relating to delivery, storage and handling of products.

## 3.2	ROUTINE MATTERS

Refer to 1250 TEMPORARY WORKS & SERVICES for protection requirements. Refer to 1270 CONSTRUCTION for requirements relating to defective or damaged work, removal of protection and cleaning.

## 3.3	PRE-INSTALLATION REQUIREMENTS

Check work previously carried out and confirm it is of the required standard for this part of the work.

FABRICATION

## 3.4	FABRICATION

Fabricate the canopy and associated products as detailed and in accordance with Entrada requirements. Do not cut, drill or modify Equinox components on site except as shown in Entrada installation documentation.

## 3.5	COLD FORMED

Ensure cold formed work is free from warping, buckling and fractures. Form bends with a brake press or by cold rolling.

## 3.6	CORNERS

Unless specified otherwise, mitre junctions of identical sections.

## 3.7	HOLES

Form without distortion of surrounding metal.

## 3.8	CLEANING DURING ASSEMBLY

Remove all burrs and sharp arrises which would be visible after fixing, or a hazard to the user. Remove any slag or spatter from the welds.

## 3.9	RIVETED JOINTS

Draw riveted joints tightly together, with rivets closed to completely fill holes.

## 3.10	MECHANICAL JOINTS

Ensure mechanical joints are tight with no gaps. Use fixings suited to purpose and as specified by Entrada.

## 3.11	PREPARATION FOR COATINGS

Before applying coatings remove all welding slag, weld spatter, anti-spatter compounds, paints, grease, flux, rust, burrs and sharp arrises. Make good all defects which would show after application of coating. Finish surfaces smooth.

INSTALLATION/APPLICATION

## 3.12	STANDARDS AND TOLERANCES

Refer to the general section 1270 CONSTRUCTION for general requirements.

## 3.13	INSTALLATION GENERALLY

Install the canopy system plumb, true to line and face, and square in accordance with Entrada installation documentation and the project-specific engineered design documentation.

## 3.14	SEPARATION

Isolate dissimilar materials and dissimilar metals in close proximity by painting the surfaces or fitting separator strips such as high density polyethylene packers.

COMPLETION & COMMISSIONING

## 3.15	COMPLETION MATTERS

Refer to 1270 CONSTRUCTION for completion requirements and if required commissioning requirements.

# 4	SELECTIONS

For further details on selections go to entradacanopies.com. Substitutions are not permitted to the following, unless stated otherwise.

CANOPY SYSTEM

## 4.1	ENTRADA EQUINOX CANTILEVERED CANOPY SYSTEM

| Location | [Location on drawings] |
| --- | --- |
| Brand | Entrada Equinox Cantilevered Canopy |
| Material | Aluminium |
| Size (width × projection) | [mm × mm] |
| Fixing method/type | [Fixing method/type] |
| Finish | [Powder coat range] powder coat |
| Colour | [Colour name] |